MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 3/02/00						DISINFECTANT & DEODORANT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: Disinfectant and Deodorant

TRADE NAME: Disinfectant and Deodorant

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 3, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	SARA III LIST	OSHA PEL	ACGIH TLV	Cardinogen Ref. Source

Ethanol				64-17-5		No		1000		1000		d

Sodium Nitrate			7632-00-0		No		N/E		N/E		d

Methyl Alcohol			67-56-1		Yes		200		200		d

Ammonium Hydroxide		1336-21-6		No		35		25		d

Isobutane/Propane blend		75-28-5		No		800		800		d

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg:  N/A

VAPOR PRESSURE: (psig @70°F Aerosols): Max 60

VAPOR PRESSURE: (Non-Aerosols)(mmm Hg and Temperature): N/A

VAPOR DENSITY: > N/E

SPECIFIC GRAVITY OR BULK DENSITY: (H2O = 1) Concentrate Only=0.840

SOLUBILITY IN WATER: Soluble

EVAPORATION RATE (BuAc = 1): N/E

APPEARANCE and ODOR: Clear, colorless spray with light, airy fragrance

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: (Non-Aerosols): N/A

FLAMMABILITY LIMITS IN AIR as per USA Flame Projection Test): Non-flammable

EXTINGUISHING MEDIA: Foam, dry chemical, carbon dioxide, water

SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: Do not expose aerosols to temperatures above 130°F or the container may rupture.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Open flame, welding arcs, heat.

CHEMICAL INCOMPATIBILITY: Avoid contact with oxidants, reducing agents, ammonium salts.

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Eye contact, Skin contact, Ingestion

HEALTH HAZARDS: Inhalation: Excessive inhalation of vapors can be harmful and may cause headaches, dizziness, asphyxia, anesthetic effects and possible unconsciousness; Eyes: Mild irritation; Skin: Possible mild irritation; Ingestion: Possible chemical pneumonitis if aspirated into lungs. Nausea.

CHRONIC EFFECTS: (Effects due to excessive exposure to the raw materials of this mixture) Overexposure may cause kidney damage, liver abnormalities, brain damage.

MEDICAL CONDITIONS AGGRAVATED: May aggravate existing eye, skin, or upper respiratory conditions.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with water for 15 minutes. If irritated, seek medical attention..

INGESTION: Drink large glasses of water. INDUCE VOMITING. Seek medical attention immediately.

INHALATION: Remove to fresh air. Resuscitate if necessary. Get medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If vapor concentration exceeds TLV, use respirator approved by NIOSH in positive pressure mode.

VENTILATION: Adequate ventilation to keep vapor concentration below TLV.

EYE PROTECTION: Safety glasses recommended.

PROTECTIVE GLOVES: Latex, if skin is easily irritated..

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Wash with soap and water before handling food. Remove contaminated clothing.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Absorb with suitable medium. Incinerate or landfill according to local, state or Federal regulations. DO NOT FLUSH TO SEWER.

WASTE DISPOSAL METHOD: Aerosol cans when vented to atmospheric pressure through normal use, pose no disposal hazard.

PRECAUTIONS IN HANDLING: Do not puncture or incinerate containers. Do not store at temperatures above 130°F.

OTHER PRECAUTIONS & SPECIAL HAZARDS: KEEP OUT OF REACH OF CHILDREN. Avoid food contamination. Avoid breathing vapors. Remove ignition sources. Do not use on polished wood furniture or rayon fabrics.

NA = Not Applicable     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.