MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/10/98							STRONG BOWL CLEANER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-103

TRADE NAME: STRONG BOWL CLEANER

SYNONYMS: NA

HMIS RATING(0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3; Flammability: 0; Reactivity: 2; Personal Protection Equipment: G

SECTION: 2					HAZARDOUS INGREDIENTS/ HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT	OSHA PEL	ACGIH TLV	SEC 313

Hydrochloric Acid			7647-01-0		40 - 60		5 ppm Ceiling	5 ppm Ceiling	Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 230°F

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): 1.72

SPECIFIC GRAVITY OR BULK DENSITY: 1.12

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): 1

APPEARANCE: Opaque, white to clear liquid

ODOR: Pungent, irritating odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water

SPECIAL FIRE FIGHTING PROCEDURES: All containers should be cooled with water to prevent vapor pressure build up. Fire fighters should wear full protective clothing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Fire fighters should wear self-contained positive pressure breathing apparatus.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with bases can cause violent reaction generating large amounts of heat.

CHEMICAL INCOMPATIBILITY: Metals, alkalis and phosphates. Do not mix with other chemicals.

HAZARDOUS DECOMPOSITION: None

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: None

OXIDIZER: NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Eyes, Ingestion

HEALTH HAZARDS: Inhalation: Hydrogen chloride gas can cause irritation of respiratory tract. Skin: Hydrogen chloride vapor can rapidly cause burning of skin. Eyes: Liquid can cause eye irritation, severe burns and permanent damage. Ingestion: May cause burns to the mouth.

CHRONIC EFFECTS OF OVER EXPOSURE: Will burn skin, eyes, throat and can cause severe irritation.

MEDICAL CONDITIONS AGGRAVATED: Asthma, bronchitis, emphysema and other lung conditions and chronic nose, sinus and throat conditions. Burns the skin and eyes.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Remove contaminated clothing. Wash thoroughly with soap and water for 15 minutes. If irritation persists, contact physician.

EYES: Flush eyes with water for 15 minutes. Get medical attention.

INGESTION: Give large amounts of water. DO NOT induce vomiting. Get a physician.

INHALATION: Move to fresh air immediately. If breathing stops, administer artificial respiration. Get medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Recommended

VENTILATION: Local exhaust: Sufficient     Mechanical: Recommended

PROTECTIVE CLOTHING: Apron and rubber shoes should be worn.

EYE PROTECTION: Goggles

PROTECTIVE GLOVES: Rubber

OTHER PRECAUTIONS: Handle with care

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Evacuate immediate area where fumes are present. Clean up personnel must wear protective equipment. Completely contain acid with dikes. Neutralize the spilled product with soda ash.

WASTE DISPOSAL METHOD: Consult Federal, State and Local disposal authorities for approved procedures.

PRECAUTIONS IN HANDLING: Store in a cool, dry place. After each use make sure container is tightly closed. Keep out of reach of children. Read all directions on label of container before using.

NA = Not Applicable     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.