MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/17/98								SUPERSORB

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-154

TRADE NAME:  SUPERSORB

SYNONYMS: Chlorinated powdered super absorbent

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Toxicity: 1, Fire: 1, Reactivity: 0, Special: E

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Amorphous Silica			1343-98-3		NA			5 mg/m3			5 mg/m3			Yes

Sodium Polyacrylate			9003-04-7		NA			NA			NA			No

Chlorinated Isocyanurals		87-90-1		NA			NA			NA			Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg:  None

pH:  pH Concentrate: X	pH Use Dilution: 7.0

VAPOR PRESSURE: None

VAPOR DENSITY: None

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 0.5 (Gravity)

SOLUBILITY IN WATER:  Negligible, less than 0.1%

EVAPORATION RATE (BuAc = 1): 0.0

APPEARANCE AND ODOR:  White granular powder, mild odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): LEL: NA	UEL: NA

EXTINGUISHING MEDIA: Water spray or fog

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained positive pressurized breathing apparatus MSHA/NIOSH approved or equivalent to maintain TLV. Use fine mist to suppress dust.

UNUSUAL FIRE & EXPLOSION HAZARDS: Fine dust may cause static reactions.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY:  Stable, store below 115°F. Store in a cool dry place

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Strong acids

HAZARDOUS DECOMPOSITION: If burned, normal combustion products: carbon dioxide, carbon monoxide, nitrous oxides, chlorine

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: NA

HEALTH HAZARDS: May cause eye, nose and skin irritation.

CARCINOGENICITY: NTP: NA	IARC Monographs: No	OSHA Regulated: Yes

CHRONIC EFFECTS OF OVER EXPOSURE: Eyes: Irritation. Inhalation: Irritation. Skin: Irritation.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Rinse with plenty of water. See physician if irritation persists.

EYES: Flush with flowing water for 15 minutes and see physician.

INGESTION: CALL A PHYSICIAN OR POISON CONTROL CENTER IMMEDIATELY. NEVER GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON.

INHALATION: Remove to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Dust mask

VENTILATION: Local exhaust preferred. Mechanical (general) at point of dust release acceptable. Avoid breathing product dust.

PROTECTIVE CLOTHING: To avoid spilling on skin and clothing

EYE PROTECTION: Goggles or safety glasses

PROTECTIVE GLOVES: Plastic or rubber

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Wash after handling.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Sweep up to suitable containers for disposal. Floors may be slippery. Use care to avoid falls.

WASTE DISPOSAL METHOD: Dispose in accordance with applicable Federal, State and Local regulations.

PRECAUTIONS IN HANDLING:  KEEP OUT OF REACH OF CHILDREN. Store upright in original closed container. Do not get in eyes, on skin or on clothing. Remove and wash contaminated clothing before reuse.

NA = Not Applicable     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.