MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: January 19, 2000								DIS CHARGE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-165

TRADE NAME: DIS CHARGE

SYNONYMS: Anti-Static

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 2, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				N/A		N/A			N/A			N/A			NO

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg:  N/A

pH: NA

VAPOR PRESSURE: (psig @70°F): N/A

VAPOR DENSITY: > 1

PERCENT VOLATILE BY WEIGHT (%): N/A

SPECIFIC GRAVITY OR BULK DENSITY: (H2O = 1) @ 70°F: N/A

SOLUBILITY IN WATER: Soluble

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE and ODOR: Clear liquid with slight odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: (of concentrate only): None

FLAMMABILITY LIMITS IN AIR as per USA Flame Projection Test): N/A

EXTINGUISHING MEDIA: Dry chemical, Water fog

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing. Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: N/A

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents.

HAZARDOUS DECOMPOSITION: N/A

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: May cause slight irritation . Skin: Frequent or prolonged contact may cause irritation. Inhalation: Inhalation of mist can cause irritation of nasal and respiratory passages. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting and diarrhea. Aspiration of material into the lungs can cause chemical pneumonitis.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. Seek medical attention.

INGESTION: Seek medical attention immediately.

INHALATION: Remove to fresh air. Seek medical attention immediately. If breathing stops give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust

PROTECTIVE CLOTHING: NA

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: None needed for proper use in accordance with label directions. Use chemical resistant gloves if hand contact will be made.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. Cover with absorbent material and sweep up. Wash area to prevent slipping. Dispose of soaked absorbent material in accordance with Federal, State and Local laws.

WASTE DISPOSAL METHOD:  Consult Federal, State and Local authorities for approved procedures.

PRECAUTIONS IN HANDLING: Keep out of reach of children. For industrial and institutional use only. Store in a cool, dry area away from heat or open flame.

NA = Not Applicable     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.