MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/10/98									DIAMOND

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-200

TRADE NAME: DIAMOND

SYNONYMS: Wash and Wax Concentrate

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 0, Reactivity: 0, Personal Protection: A

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Isopropanol			67-63-0		NA			400 ppm			400 ppm			No

Ethylene Glycol			107-21-1		NA			50 ppm			50 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg:  212°F

pH:  6 - 7

VAPOR PRESSURE: 17.5 @ 68°F

VAPOR DENSITY: Heavier than air

PERCENT VOLATILE BY WEIGHT (%): 19.52

SPECIFIC GRAVITY OR BULK DENSITY: 0.97 @ 77°F

SOLUBILITY IN WATER:  Soluble

EVAPORATION RATE (BuAc = 1): >1

APPEARANCE:  Thin yellow liquid, no odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: > 200°F

FLAMMABILITY LIMITS IN AIR (% V): UEL: 12  LEL: 2

EXTINGUISHING MEDIA: Dry chemical, water fog, alcohol foam

SPECIAL FIRE FIGHTING PROCEDURES: Use equipment suitable for ordinary combustible material. Product contains a large amount of water and would not normally burn.

UNUSUAL FIRE & EXPLOSION HAZARDS:  None known

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents

HAZARDOUS DECOMPOSITION: NA

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: NA

HEALTH HAZARDS: Skin: Prolonged or repeated contact can cause moderate irritations, defatting dermatitis. Eyes: Vapor, especially when heated, can cause irritation, redness, tearing, blurred vision; liquid can cause severe permanent eye damage. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting, diarrhea. Inhalation: High concentrations (400 ppm) of isopropanol vapors can cause drowsiness and irritation of the respiratory tract.

CHRONIC EFFECTS OF OVER EXPOSURE: It is not regulated by OSHA as a carcinogen.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Thoroughly wash exposed area with soap and water. Remove contaminated clothing and launder before re-use.

EYES: Immediately flush with large amounts of water for at least 15 minutes lifting upper and lower lids occasionally. Call a physician.

INGESTION: Immediately drink two large glasses of water. Call a physician.

INHALATION: Move to fresh air. If breathing is difficult, administer oxygen and call a physician.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Not required under normal conditions of use.

VENTILATION: Provide local exhaust

PROTECTIVE CLOTHING: NA

EYE PROTECTION: Chemical splash goggles if splashing occurs

PROTECTIVE GLOVES: None

OTHER PRECAUTION: NA

WORK/HYGIENIC PRACTICES: Wash hands with soap and water before eating, drinking, smoking or using toilet facilities.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Dike area of spill to prevent spreading and pump liquid into salvage tank. Absorb remaining liquid with floor absorbent or other absorbent material and shovel into containers. This product is toxic to fish: prevent run-off to sewers, streams or other bodies of water.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Keep in closed or covered containers. Store in cool, dry place with adequate ventilation. Exercise caution when handling contents of the container. Keep out of reach of children. Read all label directions before using.

NA = Not Available     <= Less than     >= Greater than      NE= Not Established

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.