MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: September 26, 2000								KLEAR-VIEW

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-201

TRADE NAME: KLEAR VIEW

SYNONYMS: Windshield Washer and Deicer Concentrate

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 3, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Methanol				67-56-1		55.0 - 75.0%		200 ppm			200 ppm

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT (°F): >100

VAPOR PRESSURE (mm Hg): ND

VAPOR DENSITY (Air=1): ND

SPECIFIC GRAVITY OR BULK DENSITY (Water = 1): 0.875

SOLUBILITY IN WATER:  Soluble

VOC CONTENT (% by weight): 65.0 - 70.0

pH: NA

EVAPORATION RATE (Water =1)): >1.0

APPEARANCE and ODOR:  Clear, dark green liquid; alcohol odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  <70

FLAMMABILITY LIMITS IN AIR (% V): UPPER: ND	LOWER: ND  (Test Method): Closed cup

EXTINGUISHING MEDIA: Water, foam, carbon dioxide, dry chemical.

SPECIAL FIRE FIGHTING PROCEDURES: Cool fire exposed containers with water fog. Firefighters should be equipped with full protective gear including self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS:  Vapors may cause flashback.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Excess heat and open flame.

CHEMICAL INCOMPATIBILITY: Strong oxidizers.

HAZARDOUS POLYMERIZATION: Will not occur.

HAZARDOUS DECOMPOSITION: Thermal decomposition may produce oxides of carbon.

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Eye Contact, Skin Contact, Ingestion

INGREDIENTS THAT ARE CONSIDERED BY OSHA, NTP, LARC TO BE SUSPECTED HUMAN CARCINOGENS: None

HEALTH HAZARDS: If in eyes: mild irritation and redness; If on skin: redness, dryness, irritation, defatting with prolonged or repeated exposure; If swallowed: gastrointestinal irritation, dizziness, drowsiness, drunkenness, weakness, CNS depression.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. Remove contaminated clothing and launder before reuse. If irritation persists, obtain medical attention.

EYES: Flush under eyes and eyelids with plenty of cool water for at least 15 minutes. If irritation persists, obtain medical attention.

INGESTION: Contact physician or poison control center immediately. Give affected person one to two glasses of water. Do not induce vomiting unless told to do so by a physician or poison control center. Never give anything by mouth to an unconscious person.

INHALATION: Remove person to fresh air. If breathing has stopped, administer artificial respiration. If breathing is difficult, administer oxygen.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Use with adequate ventilation. If recommended Exposure Limits are exceeded wear a NIOSH approved respirator, following manufacturer’s recommendations.

VENTILATION: Local: recommended.	Mechanical: Not required.

PROTECTIVE CLOTHING: Chemical resistant gloves.

EYE PROTECTION: Safety glasses or chemical goggles.

PROTECTIVE GLOVES: Chemical resistant gloves

OTHER PRECAUTIONS: Keep out of reach of children.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Contain spill. Ventilate area and remove all sources of ignition. Soak up spilled material with inert absorbent material and place in a properly marked closed container for proper disposal.

WASTE DISPOSAL METHOD: Consult local environmental authorities.

PRECAUTIONS IN HANDLING: Store in a cool, dry place away from heat or open flame. Keep container tightly closed when not in use.

NA = Not Available     <= Less than     >= Greater than		ND=Not Determined

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.