MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/10/98										TITANIC

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-208

TRADE NAME: TITANIC

SYNONYMS: ALUMINUM BRIGHTENER

HMIS RATING(0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3; Flammability: 0, Reactivity: 4; Personal Protective Equipment: G

SECTION: 2					HAZARDOUS INGREDIENTS/ HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	 APPROX. % WT	OSHA PEL	ACGIH TLV	SEC 313

Phosphoric Acid			7664-38-2		20		1 mg/m3		1 mg/m3		Yes

Ammonia Bilfloride			1341-49-7		3		2.5 mg/m3 as F	2.5 mg/m3 as F	Yes

& 25 ppm as NH3	& 25 ppm as NH3

2-Butoxyethanol			111-76-2		NA		50 ppm 		50 ppm		Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 180°F

pH: NA

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): 5.94

SPECIFIC GRAVITY OR BULK DENSITY: 1.2

SOLUBILITY IN WATER: Partially

EVAPORATION RATE (BuAc = 1): Low

APPEARANCE: Clear liquid

ODOR: Solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water, dry foam

SPECIAL FIRE FIGHTING PROCEDURES: All containers should be cooled with water. Fire fighters should wear protective clothing.

UNUSUAL FIRE & EXPLOSION HAZARDS:  None known

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Metals and glass

CHEMICAL INCOMPATIBILITY: Alkalis and oxidizers. Do not mix with other chemicals.

HAZARDOUS DECOMPOSITION: None known

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: Yes

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Skin, Ingestion, Inhalation, Eyes

HEALTH HAZARDS: Eyes: Can cause irritation and possible blindness. Ingestion: Can burn throat and mouth.

CHRONIC EFFECTS OF OVER EXPOSURE: Burning of the skin, mouth and throat, dizziness and nausea

MEDICAL CONDITIONS AGGRAVATED: Possible irritation to the eyes and blindness; burning of the mouth

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Remove contaminated clothing and wash with soap and water for 15 minutes.

EYES: Flush with water for 15 minutes lifting upper and lower eyelids. Call a physician.

INGESTION: Rinse mouth with water. Then drink 1 or 2 glasses of water. Never give anything by mouth to an unconscious person. Get medical attention.

INHALATION: NA

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Not recommended

VENTILATION: Local exhaust: Sufficient  	Mechanical: Not recommended

PROTECTIVE CLOTHING: Acid proof apron and shoes

EYE PROTECTION: Chemical resistant goggles

PROTECTIVE GLOVES: Water impervious (latex, neoprene)

OTHER PRECAUTIONS: Eye wash recommended

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Do not allow liquid into sewer systems or lakes by diking area. Neutralize liquid with soda ash and collect it.

WASTE DISPOSAL METHOD: Dispose of according to Federal, State and Local regulations.

PRECAUTIONS IN HANDLING: Keep lids on containers. Store in a cool, dry place. Keep out of reach of children. Read all instructions on label before using.

NA = Not Applicable     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.