MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/17/98									MR. BLUE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-211

TRADE NAME:  MR. BLUE

SYNONYMS: Instant White Wall Tire Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme):  Health: 1, Fire: 0, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Ethylene Glycol Monobutyl Ether	111-76-2		NA			50 ppm			50 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212°F

pH: Concentrate: 13	Use Dilution: 10.5

VAPOR PRESSURE: 17

VAPOR DENSITY: 0.62

PERCENT VOLATILE BY WEIGHT (%): 7.59

SPECIFIC GRAVITY OR BULK DENSITY: 1.02

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): >1.00

APPEARANCE:  Blue, thin liquid

ODOR: Butyl type odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water spray or fog, foam, dry chemical, carbon dioxide, alcohol foam

SPECIAL FIRE FIGHTING PROCEDURES: Avoid exposure to fumes or vapors. Wear self-contained positive pressurized breathing apparatus, MSHA/NIOSH approved or equivalent to maintain TLV.

UNUSUAL FIRE & EXPLOSION HAZARDS:  Use water spray to cool adjacent fire exposed containers. Product will not burn but may splatter if temperature exceeds boiling point.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY:  Stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents, acids

HAZARDOUS DECOMPOSITION: If burned, normal combustion products: carbon dioxide, carbon monoxide, nitrous oxides

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: NA

HEALTH HAZARDS: NA

CHRONIC EFFECTS OF OVER EXPOSURE: Skin: Burns. Eyes: Burns. Inhalation: Irritation.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water for 15 minutes. Seek physician if irritation persists.

EYES: Flush with flowing water for 15 minutes and see physician.

INGESTION: CALL A PHYSICIAN OR POISON CONTROL CENTER IMMEDIATELY. NEVER GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON.

INHALATION: Remove to fresh air. If breathing has stopped, apply suitable artificial respiration. Seek medical help.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  Normal room ventilation

VENTILATION: Local exhaust preferred. Mechanical (general) at point of mist acceptable. Avoid breathing vapor or spray.

PROTECTIVE CLOTHING: Apron to avoid contact on skin and clothes are recommended.

EYE PROTECTION: Safety glasses, splash proof chemical goggles when handling concentrate are recommended.

PROTECTIVE GLOVES: Recommended

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES:  Do not eat, drink or smoke in work area.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Dike area. Mop up entire contents and place in a marked container.

WASTE DISPOSAL METHOD: Dispose of in accordance with applicable Federal, State and Local regulations.

PRECAUTIONS IN HANDLING: KEEP OUT OF REACH OF CHILDREN. Avoid all contact. Store upright in original closed container. Do not get into eyes, on skin or clothing. Remove and wash contaminated clothing before reuse.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.