MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/10/98									WOW

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-223

TRADE NAME:  WOW

SYNONYMS: Heavy Duty Cleaner Degreaser

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Flammability: 0, Reactivity: 1, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Sodium Hydroxide			1310-73-2		3-5			2 mg/m3			2 mg/m3			Yes

Ethylene Glycol Mono Butyl Ether	111-76-2		10-15			25 ppm			25 ppm			Yes

SECTION: 3					PHYSICAL DATA

pH:  12

VAPOR PRESSURE: 80

VAPOR DENSITY: >1

PERCENT VOLATILE BY WEIGHT (%): 20.06

SPECIFIC GRAVITY OR BULK DENSITY: 1.060

SOLUBILITY IN WATER:  Complete

APPEARANCE:  Brown to clear liquid

ODOR: Lemon to solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  None

FLAMMABILITY LIMITS IN AIR (% V): Non flammable

EXTINGUISHING MEDIA: Foam, dry media, carbon dioxide

SPECIAL FIRE FIGHTING PROCEDURES: Wear self contained breathing apparatus and protective clothing. Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: NA

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Do not expose to temperatures above 120°F.

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents and strong acids.

HAZARDOUS DECOMPOSITION: NA

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Skin, Eyes, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: Will cause irritation and can injure eye tissue. Skin: Frequent or prolonged contact will cause irritation. Inhalation: Inhalation of mist can cause irritation of nasal and respiratory passages. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting and diarrhea. Aspiration of material into the lungs can cause chemical Pneumonitis.

CHRONIC EFFECTS OF OVER EXPOSURE: Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea, and other central nervous system effects.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, get medical attention immediately.

EYES: Flush with water for 15 minutes while holding upper and lower lids open. Seek medical attention.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Remove to fresh air. Seek medical attention immediately. If breathing stops, give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  None needed when used properly in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section 2 ingredients below acceptable limits.

PROTECTIVE CLOTHING: NA

EYE PROTECTION: Goggles

PROTECTIVE GLOVES: Rubber gloves

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Maintain local exhaust and adequate ventilation. Cover with absorbent material and sweep up. Wash area to prevent slipping. Dispose of soaked absorbent material in accordance with Federal, State and Local laws.

WASTE DISPOSAL METHOD: According to State, Federal and Local authorities

PRECAUTIONS IN HANDLING: KEEP OUT OF REACH OF CHILDREN.  For Industrial And Institutional Use Only. Read and follow label instructions.

NA = Not Applicable     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.