MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/20/98								GELLED DESCALER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-226

TRADE NAME: GELLED DESCALER

SYNONYMS: NA

HMIS RATING(0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3, Flammability: 0, Reactivity: 2, Personal Protection Equipment: G

SECTION: 2					HAZARDOUS INGREDIENTS/ HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT	OSHA PEL	ACGIH TLV	SEC 313

Hydrochloric Acid			7647-01-0		40 - 60		5 ppm Ceiling	5 ppm Ceiling	Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 230°F

PERCENT VOLATILE BY WEIGHT (%): 0.87

SPECIFIC GRAVITY OR BULK DENSITY: 1.12

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): 1

APPEARANCE: Thick, pink liquid

ODOR: Pungent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water

SPECIAL FIRE FIGHTING PROCEDURES: Containers should be cooled with water to prevent vapor pressure build up. Fire fighters should wear full protective clothing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Fire fighters should wear self-contained positive pressure breathing apparatus.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with bases can cause violent reactions generating large amounts of heat.

CHEMICAL INCOMPATIBILITY: Metals, alkalis and phosphates. Do not mix with other chemicals.

HAZARDOUS DECOMPOSITION: None

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: None

OXIDIZER: NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Eyes, Ingestion

HEALTH HAZARDS: Inhalation: Hydrogen chloride gas can cause irritations of respiratory tract. Skin: Hydrogen chloride vapor can rapidly cause burning of skin. Eyes: Liquid can cause eye irritation, severe burns and permanent damage. Ingestion: May cause burns to the mouth.

CHRONIC EFFECTS OF OVER EXPOSURE: Will burn skin, eyes, throat and can cause severe irritation.

MEDICAL CONDITIONS AGGRAVATED: Asthma, bronchitis, emphysema, other lung conditions; chronic nose, sinus, throat conditions; burns the skin and eyes

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Remove contaminated clothing. Wash thoroughly with soap and water for 15 minutes.

EYES: Wash eyes with water for 15 minutes lifting upper and lower eyelids and rotating eyeball. Seek medical attention immediately.

INGESTION: Give large amounts of water. DO NOT induce vomiting. Seek a physician immediately.

INHALATION: Move to fresh air immediately. If breathing stops, administer artificial respiration. Seek medical attention immediately.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Recommended

VENTILATION: Local exhaust: Sufficient		Mechanical: Recommended

PROTECTIVE CLOTHING: Apron and rubber shoes should be worn.

EYE PROTECTION: Goggles

PROTECTIVE GLOVES: Rubber

OTHER PRECAUTIONS: Handle with care. Try not to work with product in confined areas. Read all label directions.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Evacuate immediate area where fumes are present. Clean up personnel must wear protective equipment. Completely contain acid with dikes. Neutralize the spilled product with soda ash.

WASTE DISPOSAL METHOD: Consult Federal, State and Local disposal authorities for approved procedures.

PRECAUTIONS IN HANDLING: Store in a cool, dry place. After each use make sure container is tightly closed. Keep out of reach of children. Read all directions on label of container before using.

NA = Not Applicable     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.