MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 1/8/02									HOMOGENIZER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-228

TRADE NAME: HOMOGENIZER & SOLUBULIZER

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Flammability: 0, Reactivity: 1, Special Hazards: None

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212°F

VAPOR PRESSURE: 25-30 mm Hg

VAPOR DENSITY: >1

SPECIFIC GRAVITY OR BULK DENSITY: 1.00-1.05

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE:  Clear, straw colored liquid

ODOR: Odorless

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  Non flammable

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Dry chemical, carbon dioxide, water and fog

SPECIAL FIRE FIGHTING PROCEDURES: Do not inhale smoke, fumes or vapors. A straight stream of water may spread fire. However, spraying water from as far away as possible may cool container.

UNUSUAL FIRE & EXPLOSION HAZARDS:  Vapors are heavier than air and may travel a considerable distance to ignition and flashback. Combustible liquid: Keep away from all sources of ignition.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with ignition sources, hot glowing surfaces, spark and open flame

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents and strong acids

HAZARDOUS DECOMPOSITION: Carbon monoxide, hydrogen chloride

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: Eyes: Irritant. Skin: Irritant. Inhalation: Spray mist is irritating to the respiratory tract. Ingestion: Harmful or fatal if swallowed.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye irritation and possible burning, skin irritation, burning dermatitis, irritation to nose and throat, illness or death if ingested

MEDICAL CONDITIONS AGGRAVATED: Bronchial conditions, skin irritations and dermatitis, eye irritation

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash well with soap and water.

EYES: Flush with water for 15 minutes. Seek a physician.

INGESTION: DO NOT induce vomiting. Give 1 or 2 glasses of water. Seek medical attention immediately.

INHALATION: Move to fresh air. If breathing difficulty or discomfort occurs and/or persists, seek medical attention immediately.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None

VENTILATION: Local exhaust: Fresh air   	Mechanical: Not recommended

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: Chemical goggles

PROTECTIVE GLOVES: Impervious rubber gloves

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Remove all sources of ignition, ventilate area. Wear appropriate clothing. Collect the spill with an absorbent. Do not allow spill to enter sewers or waterways. Product is toxic to fish and wildlife.

WASTE DISPOSAL METHOD: According to all applicable environmental and pollution standards

PRECAUTIONS IN HANDLING: Keep container closed when not in use. Follow all label directions. Store in a cool, dry place away from any ignition source. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.