MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/28/98										DE-TAR

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-230

TRADE NAME: DE-TAR

SYNONYMS: Tar Remover

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 2, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 348°F

VAPOR PRESSURE: 1 mm Hg @ 14°C

VAPOR DENSITY: 4.73

PERCENT VOLATILE BY WEIGHT (%): 95-100

SPECIFIC GRAVITY OR BULK DENSITY: 0.84

SOLUBILITY IN WATER:  Insoluble

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE:  Clear liquid

ODOR: Citrus smell

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  115°F (TCC)

FLAMMABILITY LIMITS IN AIR (% V): 0.7 @ 302°F

EXTINGUISHING MEDIA: Dry chemical, carbon dioxide, foam

SPECIAL FIRE FIGHTING PROCEDURES: Do not inhale smoke, fumes or vapors. A straight stream of water may cause fire to spread. However, spraying water from as far away as possible may cool the container.

UNUSUAL FIRE & EXPLOSION HAZARDS:  Vapors are heavier than air and may travel a considerable distance to ignition and flashback. Combustible liquid: Keep away from all sources of ignition.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with ignition sources, hot glowing surfaces, spark and open flame

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents and strong acids

HAZARDOUS DECOMPOSITION: Carbon monoxide and carbon dioxide from incomplete burning.

HAZARDOUS POLYMERIZATION:  Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: Eyes: Irritant. Skin: Irritant. Inhalation: Spray mist is irritating to the respiratory tract. Ingestion: Harmful or fatal if swallowed.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye irritation and possible burning. Skin irritation, burning dermatitis. Irritation to nose and throat. Illness or death if ingested.

MEDICAL CONDITIONS AGGRAVATED: Bronchial conditions, skin irritation and dermatitis, eye irritation

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash well with soap and water. Seek physician if irritation persists.

EYES: Flush with water for 15 minutes holding upper and lower eyelids open to rinse entire eye surface. Seek a physician.

INGESTION: DO NOT induce vomiting. Seek a physician immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  None

VENTILATION: Local exhaust: Fresh air		Mechanical: Not recommended

PROTECTIVE CLOTHING: To avoid wetting of skin and clothes

EYE PROTECTION: If splashing is to occur, wear glasses.

PROTECTIVE GLOVES: If hand contact will be made, wear rubber gloves.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke when using this product. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Remove all sources of ignition; ventilate area. Wear appropriate clothing. Collect the spill with an absorbent. Do not allow spill to enter sewers or waterways. Product is toxic to fish and wildlife.

WASTE DISPOSAL METHOD: According to all applicable environmental and pollution standards.

PRECAUTIONS IN HANDLING: Keep container closed when not in use. Follow all label directions. Store in a cool, dry place away from any ignition source.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.