MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 6/12/01										SUPER CON

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-235

TRADE NAME: SUPER CON

SYNONYMS: Hand Dishwashing Liquid

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 0, Fire: 0, Reactivity: 0, Personal Protection: A

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Nonylphenoxypolyethoxyethanol	9016-45-9		NA			NE			NE			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212 °F

VAPOR PRESSURE: 17

VAPOR DENSITY: 0.62

PERCENT VOLATILE BY WEIGHT (%): 11.56

SPECIFIC GRAVITY OR BULK DENSITY: 1.026

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): >1.026

APPEARANCE: Green liquid

ODOR: Rain Forest

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water spray or fog, foam, dry chemical, carbon dioxide

SPECIAL FIRE FIGHTING PROCEDURES: Avoid exposure to fumes or vapors. Wear self-contained positive pressurized breathing apparatus MSHA/NIOSH approved or equivalent to maintain TLV.

UNUSUAL FIRE & EXPLOSION HAZARDS: Use water spray to cool adjacent fire exposed containers. Product will not burn but may splatter if temperatures exceed boiling point.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents, cationics

HAZARDOUS DECOMPOSITION: If burned, normal combustion products: carbon dioxide, carbon monoxide, nitrous oxides

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: NA

HEALTH HAZARDS: NA

CHRONIC EFFECTS OF OVER EXPOSURE: Skin: Prolonged contact can cause irritation. Eyes: Irritation, redness.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water for 15 minutes. See a physician if irritation persists.

EYES: Flush with flowing water for 15 minutes. If irritation persists, seek a physician.

INGESTION: CALL A PHYSICIAN OR POISON CONTROL CENTER IMMEDIATELY. NEVER GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON.

INHALATION: Remove to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None

VENTILATION: Normal room ventilation

PROTECTIVE CLOTHING: None

EYE PROTECTION: Safety glasses are needed if splashing occurs.

PROTECTIVE GLOVES: None

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Mop up and flush to sewer with plenty of water. Floors may be slippery. Use care to avoid falls.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: KEEP OUT OF REACH OF CHILDREN. USE CAUTION. Store upright in original closed container. Do not get in eyes, on skin or clothing.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.