MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/20/98									ASSAULT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-236

TRADE NAME: ASSAULT

SYNONYMS: Granulated Machine Dishwash

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Flammability: 0, Reactivity: 2, Personal Protection: O

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Sodium Dichloroisocynurate Dihydrate	NA		2-3%			NA			NA			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH:  NA

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: Approximately 65 pounds per cubic foot

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE:  White granular solid

ODOR: Slight chlorine

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Material does not support combustion, but may generate chlorine gas if involved in a fire.

SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus to protect against chlorine fumes

UNUSUAL FIRE & EXPLOSION HAZARDS:  Chlorine gas generations at elevated temperatures

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with acids or their solutions, contact with soft metals (aluminum), high heat, contact with organic compounds

CHEMICAL INCOMPATIBILITY: Acids, heat

HAZARDOUS DECOMPOSITION: Carbon dioxide, chlorine gas

HAZARDOUS POLYMERIZATION:  Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation

HEALTH HAZARDS: Eyes: Irritant. Skin: Irritant. Solutions which generate chlorine and high pH, may cause burns.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye or skin irritant or burning, respiratory irritation.

MEDICAL CONDITIONS AGGRAVATED: Eye irritation, skin irritation and dermatitis; bronchial conditions.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water.

EYES: Rinse with copious amounts of water, seek physician.

INGESTION: Drink large amounts of water. DO NOT induce vomiting. Call a physician.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: To avoid breathing dusts or generated chlorine gas.

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: To avoid skin and clothing contact

EYE PROTECTION: Recommended

PROTECTIVE GLOVES: Recommended

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Sweep material up and return to container or discard. Wash away residue with water. Keep out of natural waterways and public water systems.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Keep out of reach of children. Keep container closed when not in use. Protect from moisture. Store in cool, dry area. Read entire label for complete cautions and directions. Solutions are alkaline avoid all skin and eye contact. Use good housekeeping procedures and common sense as with any alkaline cleaner. Do not mix with any other cleaners or chemicals.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.