MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/20/98			LIQUID MACHINE DISHWASHING COMPOUND

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-238

TRADE NAME: LIQUID MACHINE DISHWASHING COMPOUND

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme):  Health: 3, Flammability: 0, Reactivity: 3, Special Hazard: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Potassium Hydroxide		1310-58-3		3-5%			2 mg/m3 (ceil)		2 mg/m3 (ceil)		No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212°F

pH: NA

VAPOR PRESSURE: 25-30 mm Hg

VAPOR DENSITY: >1

PERCENT VOLATILE BY WEIGHT (%): 6.03

SPECIFIC GRAVITY OR BULK DENSITY: 1.15

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE:  Rust colored liquid

ODOR: NA

SECTION 4					FIRE AND EXPLOSION DATA

EXTINGUISHING MEDIA: Material will not support combustion.

SPECIAL FIRE FIGHTING PROCEDURES: None. Water may be used to cool containers. Material is corrosive. Use proper protective clothing in case of drum rupture.

UNUSUAL FIRE & EXPLOSION HAZARDS: None

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with acids, acid solutions, and soft metals, such as aluminum

CHEMICAL INCOMPATIBILITY: Acids

HAZARDOUS DECOMPOSITION: None known - heat generation upon acid contact

HAZARDOUS POLYMERIZATION:  Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Skin

HEALTH HAZARDS: Skin: Corrosive. Contact leads to dermatitis and chemical burns. Eyes: Corrosive. Contact leads to possible eye damage. Ingestion: Harmful or fatal if swallowed.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye and skin irritations and burns; burns upon ingestion

MEDICAL CONDITIONS AGGRAVATED: Dermatitis. Material is corrosive and rapidly attacks living tissue.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash skin thoroughly for 15 minutes. Call a physician if irritation persists.

EYES: Wash eyes with copious quantities of water for 15 minutes; call physician immediately. Remove contaminated clothing immediately; wash before reuse.

INGESTION: Dilute by drinking large amounts of water or fruit juice. Call a physician immediately. Remove contaminated clothing immediately; wash before reuse.

INHALATION: NA

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Fresh air

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: Eye glasses or face shield

PROTECTIVE GLOVES: Rubber

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Dike the area and collect the spill. Small spills may be collected with an absorbent. Flush away residue with water. Keep spill out of natural waterways and public drinking supplies. If desired, spill may be neutralized with a weak acid solution.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Keep out of reach of children. Keep container closed when not in use. Store in a cool place. Protect from freezing. Read all label cautions and directions before use. Do not mix with any other chemicals or cleaners. Store away from acid products. Material is alkaline and corrosive. Use common sense and good housekeeping practices in the handling of the material. Avoid all skin, eye and clothing contact.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.