MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/21/98									RINSE ADDITIVE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-240

TRADE NAME: RINSE ADDITIVE

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Flammability: 2, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212°F

VAPOR PRESSURE: 25-30 mm Hg

VAPOR DENSITY: >1

PERCENT VOLATILE BY WEIGHT (%): 39.66

SPECIFIC GRAVITY OR BULK DENSITY: 1.00 - 1.05

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE:  Thick, blue liquid

ODOR: Slight, bland odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  120°F (TCC)

EXTINGUISHING MEDIA: Material will not support combustion. Use media suitable for surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Containers involved in a fire may be cooled by applying flooding quantities of water from as far a distance as possible. Do not spread flames with water used for cooling. Vapors from the product may ignite, but solution will not support combustion. Wear appropriate fire fighting equipment, including self-contained breathing apparatus. Do not inhale smoke, fumes, vapors or combustion products.

UNUSUAL FIRE & EXPLOSION HAZARDS:  Overheated containers may rupture or melt. Vapors from this product are heavier than air and may travel a considerable distance to ignition and flashback.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Vapor ignition sources such as flames, electric arcs, high heat, sparks and hot glowing metals

CHEMICAL INCOMPATIBILITY: Strong oxidizers

HAZARDOUS DECOMPOSITION: Carbon dioxide and carbon monoxide

HAZARDOUS POLYMERIZATION:  Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Skin

HEALTH HAZARDS: Eyes: Irritant. Skin: Irritant or dermatitis. Ingestion: Harmful, causes illness. Inhalation: Possible irritation.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye irritation, skin irritation and dryness, nasal irritation

MEDICAL CONDITIONS AGGRAVATED: Eye irritation, dermatitis, respiratory irritation

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash away with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes. Seek a physician.

INGESTION: Seek a physician immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Usually not required

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: Glasses

PROTECTIVE GLOVES: Rubber

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Ventilate the area. Remove ignition sources. Wear appropriate safety equipment and attempt to stop the flow of material. Contain the spill and collect with an absorbent or by wet vacuuming. Rinse away residues with water to avoid slippery conditions. Keep gross spills out of sewers, water sources and waterways.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities. Material is biodegradable. Dispose of empty container according to RCRA regulations.

PRECAUTIONS IN HANDLING: Keep out of reach of children. Keep container closed when not in use. Read all cautions and directions before using the product. Use only as specified on the label. Do not mix with other cleaners or chemicals. Avoid all skin and eye contact.  Avoid inhalation of vapors. Store in a cool area away from ignition sources. Avoid contact with material from broken or leaking containers. Contaminated clothing must be washed before reuse. Use common sense and normal safety precautions when using this material.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.