Latest Revision Date: 7/20/98										RESTORE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-241

TRADE NAME: RESTORE

SYNONYMS: Cream Cleaner

HMIS RATING(0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Flammability: 0, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/ HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	% WT		TLV-TWA			PEL		SEC 313

Silica (No respirable dust)		14808-60-7	>20%		0.1 mg/m3			0.1 mg/m3	NA

Dodecylbenzylsufonic Acid		27176-87-0	5-10		NA				NA		NA

Nonoxynal			9016-45-9		5-10		NA				NA		NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT: ~212°F

pH(conc.): 1.4-2.0

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): <50%

SPECIFIC GRAVITY OR BULK DENSITY(H2O = 1): 1.250

SOLUBILITY IN WATER: Insoluble

EVAPORATION RATE (H2O = 1): ~1

APPEARANCE AND ODOR: White creme, mint odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water, CO2, foam or media suitable for surrounding fire

SPECIAL FIRE FIGHTING PROCEDURES: Standard fire fighting procedures may be followed, including full protective gear, NIOSH approved self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: Not known

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: None known

CHEMICAL INCOMPATIBILITY: Do not mix with other chemicals

HAZARDOUS DECOMPOSITION: Thermal decomposition on burning may produce toxic vapors or gases.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Skin, Ingestion

HEALTH HAZARDS: Ingestion: Harmful if swallowed. Eyes: Irritant. May cause permanent damage, especially to eyes. Skin: Irritant.

CHRONIC EFFECTS OF OVER EXPOSURE: None known

MEDICAL CONDITIONS AGGRAVATED: May aggravate skin disorders or respiratory ailments.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Flush with water for 15 minutes. Remove and wash contaminated clothing.

EYES: Flush immediately with water for 15 minutes. Get medical attention.

INGESTION: Rinse mouth with water. Then drink 1- 2 glasses of water. Never give anything by mouth to an unconscious person. Get immediate medical attention.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  Not expected to be necessary under normal conditions of use. Where exposure cannot be adequately controlled by general or local ventilation, use appropriate respiratory protection to prevent over exposure. An approved dust/mist respirator would be recommended.

VENTILATION: Local exhaust: As necessary	Mechanical: As adequate

PROTECTIVE CLOTHING: Not required

EYE PROTECTION: Chemical resistant goggles or face shield

PROTECTIVE GLOVES: Water impervious (latex, neoprene)

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Follow OSHA mandated procedures. Never eat or smoke while working with chemicals. Good housekeeping practices apply. Wash thoroughly after handling.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Wear appropriate protective equipment. If possible, contain spill and collect in approved container. Rinse thoroughly with water. Keep product out of sewers or waterways. Hold for disposal.

WASTE DISPOSAL METHOD: Dispose of in accordance with Federal, State and Local regulations. If possible, triple rinse container and recycle.

PRECAUTIONS IN HANDLING: Avoid contact with skin or eyes. Avoid breathing vapor, spray or mist. Store in original container, in cool dry place with adequate ventilation. Keep out of reach of children. Keep container tightly closed when not in use. Use only in accordance with label directions. If unsure about safe use, contact your supervisor immediately.

NA= Not Applicable     <=Less than     >=Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.