MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/23/98										REQUEST

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-243

TRADE NAME: REQUEST

SYNONYMS: Glass Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 2, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Isopropyl alcohol			67-63-0		15 - 30			400 ppm			400 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: 140@130°F

VAPOR DENSITY: >1

PERCENT VOLATILE BY WEIGHT (%): 15.35

SPECIFIC GRAVITY OR BULK DENSITY: <1

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE AND ODOR:  Green, water-thin liquid with Alcohol smell

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): LEL: 1.8	UEL: 9.2

EXTINGUISHING MEDIA: Carbon dioxide, foam and/or dry chemical may be used.

SPECIAL FIRE FIGHTING PROCEDURES: Containers should be cooled with water to prevent vapor pressure build up. Use equipment for shielding, as required, to protect personnel from bursting, rupturing or venting containers.

UNUSUAL FIRE & EXPLOSION HAZARDS:  At elevated temperatures (over 54°C/130°F), container exposed to direct flame or heat contact should be cooled with water to prevent weakening of container structure.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Keep away from heat, direct sunlight, open flames and sparks.

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents, heat and open flame

HAZARDOUS DECOMPOSITION: Carbon monoxide, carbon dioxide and additional toxic chemicals may be formed in small amounts.

HAZARDOUS POLYMERIZATION:  NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Eyes, Skin, Ingestion

HEALTH HAZARDS: Inhalation: Respiratory irritation. High concentrations may cause dizziness and nausea. Skin: Possible absorption. Eyes: May cause burning and irritation.

CHRONIC EFFECTS OF OVER EXPOSURE: Prolonged or repeated skin contact may lead to drying, irritation and dermatitis.

MEDICAL CONDITIONS AGGRAVATED: Pre-existing skin or eye disorders may be aggravated by this product.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Remove contaminated clothing. Flush skin with water, follow by washing with soap and water. If irritation occurs, seek medical attention. Do not reuse clothing until cleaned.

EYES: Flush eyes with plenty of water for 15 minutes while holding eyelids open. Get prompt medical attention.

INGESTION: Contact local poison control center or physician immediately.

INHALATION: Remove victim to fresh air and provide oxygen if breathing is difficult. Give artificial respiration if not breathing.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None is required if good ventilation is maintained. If exposure exceeds occupational exposure limits, use a NIOSH approved respirator to prevent over exposure.

VENTILATION: Local exhaust is adequate under normal conditions, mechanical ventilation is optional.

PROTECTIVE CLOTHING: Wear impervious clothing to prevent skin contact.

EYE PROTECTION: Chemical safety glasses are recommended.

PROTECTIVE GLOVES: Chemical resistant gloves

OTHER PRECAUTIONS: Ensure strict sanitary conditions are conformed to when working around chemicals.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Remove all sources of ignition and ventilate area. Soak up spill with an inert absorbent and place into designated disposal container. Consult local regulatory agency for proper disposition of material.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Avoid breathing vapor. Keep away from heat and flame. Use with adequate ventilation. Do not expose to direct sunlight or store at temperatures above 130°F (54°C). Please read and follow the directions on the product label; they are your best guide to using this product in the most effective way.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.