Latest Revision Date: 7/20/98				WATER-BASED PARTS WASHER CONCENTRATE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-249

TRADE NAME: WATER-BASED PARTS WASHER CONCENTRATE

SYNONYMS: All Purpose Cleaner and Degreaser

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 0, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg:  212°F

pH: NA

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): 5.97

SPECIFIC GRAVITY OR BULK DENSITY: NA

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE:  Light orange liquid

ODOR: Orange odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  200°F T.O.C.

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water or dry foam

SPECIAL FIRE FIGHTING PROCEDURES: The use of self contained breathing apparatus is recommended for fire fighters.

UNUSUAL FIRE & EXPLOSION HAZARDS:  All containers should be cooled with water to prevent rupture.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Acid

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents

HAZARDOUS DECOMPOSITION: Thermal decomposition in presence of air may yield carbon dioxide.

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: No

OXIDIZER:  NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Skin, Ingestion

HEALTH HAZARDS: Eyes: May cause irritation. Ingestion: Can cause gastrointestinal irritation.

CHRONIC EFFECTS OF OVER EXPOSURE: None

MEDICAL CONDITIONS AGGRAVATED: None

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with water rotating eyeball for at least 15 minutes. If irritation persists, seek medical attention.

INGESTION: Seek a physician immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None necessary under normal conditions.

VENTILATION: Local exhaust: None necessary	Mechanical: Normal ventilation

PROTECTIVE CLOTHING: None

EYE PROTECTION: Recommended

PROTECTIVE GLOVES: If skin contact is to be made, wear rubber or neoprene gloves.

OTHER PRECAUTIONS: None

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Use suitable absorbent and put into container.

WASTE DISPOSAL METHOD: According to Federal, State and Local regulations

PRECAUTIONS IN HANDLING: Keep in a cool, dry area, above freezing temperatures. Keep out of reach of children. Read and follow all directions.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.