MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency # 1-800-535-5053				1700 SOUTH FOURTH STREET

Information # (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 1/19/99									RUST-BE-GONE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-252

TRADE NAME: RUST-BE-GONE

SYNONYMS: RUST CONVERTER

HMIS RATING (0: Minimal; 1: Slight; 2: Moderate; 3: Serious; 4: Extreme): Health: 1; Flammability: 0, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS / HAZARD DATA

CHEMICAL NAME(S)			CAS NUMBER	% WT		TLV-TWA			PEL		SEC.313

Diethylene Glycol Mono Methyl Ether		111-77-3		NA		NA				NA		Yes

Tannic Acid				1401-55-4		<2.00		10 ppm				10 ppm		No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @760 mm Hg: 100°C

pH: 1-2

VAPOR PRESSURE PSIG @ 70°F (Aerosols): NA

VAPOR DENSITY (Air = 1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY (H2O = 1): 1.36

SOLUBILITY IN WATER: Dispersible

EVAPORATION RATE (BuAc = 1): 1.0

APPEARANCE: Off-white, opaque, slightly viscous liquid

ODOR: Bland

SECTION 4					FIRE AND EXPLOSION HAZARD DATA

FLASH POINT: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA:  Non-combustible. Use media appropriate for surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Water may be used to cool closed containers to prevent pressure buildup. Self-contained breathing apparatus with full face-piece and protective clothing should be used if involved in a fire with other materials.

UNUSUAL FIRE & EXPLOSION HAZARDS: This material will not support combustion unless the water has evaporated.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions to Avoid: None known

CHEMICAL INCOMPATIBILITY: Materials reactive to water or acids, such as alkaline materials or active metals.

HAZARDOUS DECOMPOSITION: Oxides of carbon, chlorine, hydrogen chloride, possible trace amounts of carbonyl chloride, unidentified organic compounds.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Skin, Eyes

CHRONIC EFFECTS: None known

ACUTE EFFECTS: Eyes: Irritating. Ingestion: May cause distress, but probably not toxic. Skin: Short contact will probably not cause irritation, but prolonged or repeated contact may. Inhalation: Spray or mist can irritate respiratory passages.

MEDICAL CONDITIONS AGGRAVATED: None known

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN:  Wash with soap and water. If irritation persists, get medical attention.

EYES: Flush with water for  15 minutes. If irritation persists, get medical attention.

INGESTION: DO NOT INDUCE VOMITING. Drink 3 to 4 glasses of water. Get immediate medical attention. Never give anything by mouth to an unconscious person.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None required under normal conditions of product storage and use.

VENTILATION: No special ventilation is required under normal conditions of storage and use.

PROTECTIVE CLOTHING: Clothing as required to minimize skin contact.

EYE PROTECTION: Safety glasses or chemical goggles

PROTECTIVE GLOVES: Plastic or rubber if desired

OTHER PRECAUTIONS: Emergency shower and eye wash station should be available.

WORK / HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands after handling.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Wear skin and eye protection during cleanup. Soak up liquid with absorbent and shovel into waste container. Cover and label container and remove from work area. Wash residue from spill area with water and flush to sewer serviced by a waste water treatment facility.

WASTE DISPOSAL METHOD: Consult Federal, State and Local regulations. Because its pH is 1-2, discarded material is a hazardous waste.

PRECAUTIONS IN HANDLING / STORING: Store in original shipping containers. Keep closed when not in use. Shelf life is 1 year. Protect from extreme heat and cold. Product may separate on standing - shake or stir before using. Do not get into the eyes. KEEP OUT OF REACH OF CHILDREN.  Read and follow all label directions. Do not take internally. Do not get in eyes..

NA= Not Applicable     <=Less than     >=Greater than     ~ Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.