Latest Revision Date: 7/22/98					TANK SAVER AND PUMP LUBRICATOR

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-402

TRADE NAME: TANK SAVER AND PUMP LUBRICATOR

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): NA

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): 100

SPECIFIC GRAVITY OR BULK DENSITY: NA

SOLUBILITY IN WATER: Partial

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Clear to dark liquid

ODOR: Petroleum odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 130 °F

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Dry chemical, carbon dioxide, water and fog

SPECIAL FIRE FIGHTING PROCEDURES: Do not inhale smoke, vapor or fumes. A straight stream of water may cause fire to spread. However, spraying water from as far away as possible may cool the container.

UNUSUAL FIRE & EXPLOSION HAZARDS: Vapors are heavier than air and may travel a considerable distance to ignition and flashback. Combustible liquid: Keep away from all ignition sources.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with ignition sources, hot glowing surfaces, spark and open flame.

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents and strong acids

HAZARDOUS DECOMPOSITION: Carbon monoxide, hydrogen chloride

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: Eyes: Irritant. Skin: Irritant. Inhalation: Spray mist is irritating to the respiratory tract. Ingestion: Harmful or fatal if swallowed.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye irritation and possible burning, skin irritation, burning dermatitis, irritation to the nose and throat. Illness or death if ingested.

MEDICAL CONDITIONS AGGRAVATED: Bronchial conditions, skin irritation and dermatitis, eye irritation

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. If irritation persists, obtain medical attention.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None

VENTILATION: Local exhaust: Fresh air		Mechanical: Not recommended

PROTECTIVE CLOTHING: To avoid wetting of skin and clothes

EYE PROTECTION: Glasses

PROTECTIVE GLOVES: Rubber

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Remove all sources of ignition, ventilate area. Wear appropriate clothing. Collect the spill with an absorbent. Do not allow spill to enter sewers or waterways. Product is toxic to fish and wildlife.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Keep container closed when not in use. Follow all label directions. Store in a cool, dry place away from any ignition sources.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.