Latest Revision Date: 7/11/01											HARD CURE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-456

TRADE NAME: HARD CURE

SYNONYMS: Concrete Sealer

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 3, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

1,2,4-Trimethylbenzene		95-63-6		<24			25 ppm			25 ppm			Yes

Xylene				1330-20-7		<3			100 ppm			100 ppm			Yes

Light Aromatic Naptha		64742-95-6	<17			NA			100			No

Aliphatic Naptha			64742-88-7	>5			100 ppm			100 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT: 326°F

pH: NA

VAPOR PRESSURE: 4.55

VAPOR DENSITY (AIR=1): 4.71

PERCENT VOLATILE BY WEIGHT (%): 75

SPECIFIC GRAVITY OR BULK DENSITY: 0.88

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (BuAc = 1): 3.3

MELTING POINT: NA

APPEARANCE and ODOR: Clear color, strong aromatic solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 108 °F

FLAMMABILITY LIMITS IN AIR (% V): LEL: 1.0	UEL: 6.0

EXTINGUISHING MEDIA: Use foam, CO2, or dry chemical fighting apparatus.

SPECIAL FIRE FIGHTING PROCEDURES: Use self-contained breathing apparatus. Water may be unsuitable as an extinguishing media, but helpful in keeping adjacent containers cool. Avoid spreading burning liquid with water used for cooling purposes.

UNUSUAL FIRE & EXPLOSION HAZARDS: NA

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Heat, sparks, flame and other sources of ignition.

CHEMICAL INCOMPATIBILITY: Avoid strong oxidizing agents, strong acids or selected amines.

HAZARDOUS DECOMPOSITION: Carbon monoxide and carbon dioxide if burned with insufficient air.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion, Eye Contact

HEALTH HAZARDS: Skin: Prolonged and repeated contact may cause irritation. Ingestion: Irritation of mucous membranes of the mouth, throat, esophagus and stomach. Eyes: May be an irritant. Inhalation: Exposure to mists or spray may cause respiratory tract irritation. High concentrations may cause nausea, vomiting and possible central nervous system depression.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and large quantities of water. If irritation persists, seek a physician.

EYES: Flush with large quantities of water for at least 15 minutes. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: If breathing difficulties, dizziness or light-headedness occur when working in areas with high vapor concentration, seek air free of vapors. If breathing stops, begin artificial respiration and seek immediate medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If TLV is exceeded, use self-contained NIOSH approved cartridge respirator or breathing apparatus.

VENTILATION: Local exhaust: May be required if specified time weighted TLV ranges are exceeded. MECHANICAL: Required if time weighted TLV ranges are exceeded.

EYE PROTECTION: Safety glasses, chemical goggles or face shield

PROTECTIVE GLOVES: Impermeable

WORK/HYGIENIC PRACTICES: Eye wash stations and safety showers are recommended in work areas.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Remove all possible ignition sources. Avoid breathing vapors. Provide adequate ventilation. Absorb and dispose of in accordance with local applicable regulations. Call emergency number if spillage poses threat to man or environment.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities. Use qualified disposal company to incinerate at an approved facility. Do not incinerate closed containers.

PRECAUTIONS IN HANDLING: Keep containers cool, dry and away from sources of ignition. Use and store with adequate ventilation. Avoid inhalation of vapors. Avoid personal contact with the product. If contact is made, remove saturated clothing and flush affected areas with water.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.