MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: March 27, 2000								OUT-DO

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-457

TRADE NAME: Out-Do

SYNONYMS: Acrylic Resin Emulsion

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV

Acrylic Polymer			25153-46-2	<20			N/E			N/E

Propylene Glycol Phenyl Ether		770-35-4		<4			N/E			N/E

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212°

pH: 9.0

VAPOR PRESSURE: 77°F

VAPOR DENSITY (Air=1): 5.3

PERCENT VOLATILE BY VOLUME: 80

SPECIFIC GRAVITY OR BULK DENSITY: 1.01

SOLUBILITY IN WATER: Yes

EVAPORATION RATE (BuAc = 1): 8.41#/gallon

APPEARANCE: White liquid, dries clear

ODOR: Mild

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: N/A

FLAMMABILITY LIMITS: N/A

EXTINGUISHING MEDIA: Use foam, CO2, or dry chemical fighting apparatus, and/or water fog.

SPECIAL FIRE FIGHTING PROCEDURES: Use self-contained breathing apparatus. Containers exposed to fire should be kept cool with water spray.

UNUSUAL FIRE & EXPLOSION HAZARDS: None

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Freezing

CHEMICAL INCOMPATIBILITY: Avoid strong oxidizing agents, strong acids or selected amines.

HAZARDOUS DECOMPOSITION: Carbon monoxide and carbon dioxide if burned with insufficient air.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion, Eye contact

HEALTH HAZARDS:

CHRONIC EFFECTS OF OVER EXPOSURE:No known acute effects; no known chronic effects

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. Seek attention if irritation from contact persists.

EYES: Flush eyes with large quantities of water and seek  medical attention.

INHALATION: If breathing difficulties, dizziness, or lightheadedness occur when working in areas with high vapor concentration, seek air free of vapors. If breathing stops, begin artificial respiration and seek immediate medical attention.

INGESTION: Do not induce vomiting. Seek immediate medical advice and/or attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Unless air monitoring demonstrates vapor/mist levels above applicable limits, no respirator is required. If respirator is required, the appropriate, properly fitted respirator should be worn during application. Follow  respirator manufacturer’s directions for respirator use.

VENTILATION: Required for handling in a confined area, ventilation equipment should be explosion proof.

PROTECTIVE CLOTHING: Neoprene or plastic apron and protective clothing covering exposed skin areas.

EYE PROTECTION: Minimum of safety glasses.

PROTECTIVE GLOVES: Neoprene or other nonporous.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Ventilate area. Avoid breathing of vapors. Use self-contained breathing apparatus or air masks for a spill in a confined area. Avoid all personal contact.

WASTE DISPOSAL METHOD: Dispose in chemical disposal area or in a manner that complies with local, state and federal regulations. Do not incinerate closed containers.

PRECAUTIONS IN HANDLINGAND STORAGE: Keep containers cool, dry, and away from sources of ignition. Use and store with adequate ventilation. Keep from freezing.

OTHER PRECAUTIONS: Avoid inhalation of vapors. Avoid personal contact with the product. If contact is made, remove saturated clothing and flush affected areas with water.

NA = Not Applicable		N/E = Not Established	<= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.