Latest Revision Date: 7/23/98									ACRYLIC SEALER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER:CRC-458

TRADE NAME: ACRYLIC SEALER

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): NA

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Acrylic Polymer			NA		40-60			NA			NA			No

Ethylene Glycol			107-21-1		10-20			50 ppm			50 ppm			No

Butyl Carbitol			112-34-5		10-20			50 ppm			50 ppm			No

Dibutyl Phthalate			84-74-2		10-20			5 mg/m3			5 mg/m3			No

Tributoxy Ethyl Phosphate		NA		5-10			NA			NA			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: Variable

VAPOR PRESSURE: >16 @ 68 °F

VAPOR DENSITY (Air=1): <1

PERCENT VOLATILE BY WEIGHT (%): 6.71

SPECIFIC GRAVITY OR BULK DENSITY: ~1

SOLUBILITY IN WATER: Dilutable

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE: Milky white liquid

ODOR: Mild acrylic odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: Non-combustible

EXTINGUISHING MEDIA: NA

SPECIAL FIRE FIGHTING PROCEDURES: NA

UNUSUAL FIRE & EXPLOSION HAZARDS: Material may splatter when above 100 °C/212 °F. Polymer film can burn.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Temperatures above 177 °/350 °F

CHEMICAL INCOMPATIBILITY: Oxidizing agents

HAZARDOUS DECOMPOSITION: NA

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation

HEALTH HAZARDS: NA

CHRONIC EFFECTS OF OVER EXPOSURE: Inhalation: Vapor or mist can irritate nose and throat and cause narcosis, nausea, vomiting, headache, drowsiness, slurred speech, dizziness, stupor and unconsciousness. Eyes: Substantially irritating to eyes. Skin: Slightly irritating to skin, can be absorbed through intact skin in harmful amounts. Overexposure to solvents by any route may cause liver or kidney damage.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritation persists.

INGESTION: Dilute by giving 2 glasses of water to drink. Seek medical attention immediately. Never give anything by mouth to an unconscious person.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Wear suitable respirator (MSHA/NIOSH approved) where exposure limits are exceeded.

VENTILATION: Mechanical local exhaust ventilation at point of contaminate release.

EYE PROTECTION: Chemical splash proof glasses

PROTECTIVE GLOVES: Impervious

OTHER PRECAUTIONS: Eye wash station and safety shower

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Dike and contain spill with inert material (e.g. sand, earth). Transfer liquid to containers for recovery or disposal and solid diking material to separate containers for disposal. Remove contaminated clothing promptly and wash affected skin areas with soap and water. Wash clothing before reuse. Keep spills and cleaning runoffs out of municipal sewers and open bodies of water.

WASTE DISPOSAL METHOD: Incinerate liquid and contaminated diking material at a permitted facility in accordance with Federal, State and Local authorities.

PRECAUTIONS IN HANDLING: Keep from freezing. Product may coagulate. Keep container closed when not using. Follow all label directions. Store in a cool, dry place. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.