MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 12/09/02										REVISE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-463

TRADE NAME:  REVISE

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Flammability: 0, Reactivity: 1, Special Hazard: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Poly(oxy-1,2-ehtanediyl), alpha-

(nonylohenyl-omega-hydroxy)	009016-45-9	1-5			2 mg/m3			2 mg/m3			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212°F

pH:  NA

VAPOR PRESSURE: 25-30 mm Hg

VAPOR DENSITY: >1

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 1.00 - 1.05

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE:  Yellow liquid

ODOR: Pine odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Material will not support combustion; use media suitable for surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Cool containers involved in a fire by applying flooding quantities of water from as far a distance as possible. Do not spread flames with water used for cooling purposes. Wear appropriate safety equipment. Do not inhale smoke, fumes, vapors or combustion products.

UNUSUAL FIRE & EXPLOSION HAZARDS: Over heated containers may rupture or melt.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: High heat, contact with acids and/or bases

CHEMICAL INCOMPATIBILITY: Acids, bases, strong oxidizing agents

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Eyes, Ingestion

HEALTH HAZARDS: Eyes: Minimal to moderately irritating to eyes.  Skin: Frequent or prolonged contact may irritate and cause redness and mild discomfort. Inhalation: May cause irritation of the nose and throat, headache, nausea, and drowsiness.  Ingestion: Abdominal discomfort, nausea, and diarrhea may occur.

CHRONIC EFFECTS OF OVER EXPOSURE: None known

MEDICAL CONDITIONS AGGRAVATED: Dermatitis

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash off with soap and water. Avoid contact with open wounds or broken skin.

EYES: Flush well with water for at least 15 minutes holding upper and lower eyelids open. Seek a physician immediately.

INGESTION: DO NOT induce vomiting. Dilute by drinking water. Seek a physician immediately.

INHALATION: Move to fresh air. If breathing is difficult or discomfort occurs and/or persists, seek medical attention immediately.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  Usually not required

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: If splashing is to occur, wear safety glasses.

PROTECTIVE GLOVES: If hand contact is to be made, wear rubber gloves.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, smoke or drink when using this product. Wash hands after  use.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Contain spill. Attempt to stop the flow of material. Collect with an absorbent or by wet vacuuming. Rinse away residues to avoid slippery conditions. Keep large spills out of sewers, water systems and water ways.

WASTE DISPOSAL METHOD: According to all applicable Federal, State and Local regulations. Dispose of empty containers according to RCRA regulations.

PRECAUTIONS IN HANDLING: Keep out of reach of children. Keep container closed when not in use. Read and follow all label cautions and directions. Use only as specified on the label. Do not mix with other cleaners or chemicals. Avoid all skin and eye contact.  Keep from freezing.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.