MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/30/98									DUST NO MORE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-464

TRADE NAME: DUST NO MORE

SYNONYMS: Sweeping Compound

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 0, Fire: 1, Reactivity: 0, Special Hazard: None

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: NA

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: ~6.66 lbs/gal packed

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Brown color

ODOR: Slight petroleum odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Normal agents for extinguishing wood fires: water spray, carbon dioxide and dry chemical.

SPECIAL FIRE FIGHTING PROCEDURES: Material will support combustion. May be drenched with water to reduce fire hazard. Material may smolder after it is involved in a fire. Spread material out and be sure fire is extinguished.

UNUSUAL FIRE & EXPLOSION HAZARDS: None

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Ignition sources

CHEMICAL INCOMPATIBILITY: Strong oxidizers

HAZARDOUS DECOMPOSITION: Carbon dioxide and carbon monoxide if burned

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: NA

HEALTH HAZARDS: Eyes: Irritant. Skin: Possible irritant especially for sensitive individuals.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye irritation, skin irritation or dermatitis

MEDICAL CONDITIONS AGGRAVATED: Dermatitis

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritation persists.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Usually not required

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: None

EYE PROTECTION: Generally not required

PROTECTIVE GLOVES: Recommended for persons sensitive to petroleum.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly after using or handling this product.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Sweep up material and return to container or discard.

WASTE DISPOSAL METHOD: Material may be dumped but follow local pollution ordinances.

PRECAUTIONS IN HANDLING: Keep out of the reach of children. Keep container closed when not in use. Store in a cool, dry place. Use normal good housekeeping practices. Read label cautions and directions before use.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.