MATERIAL SAFETY DATA SHEET					COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053						1700 SOUTH FOURTH STREET

Information #: (402) 379-0100						NORFOLK, NE 68701

Latest Revision Date: 7/23/98										ATTRACT-O-DUST

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-466

TRADE NAME: ATTRACT-O-DUST

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): NA

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~395 °F (initial)

pH: Essentially neutral

VAPOR PRESSURE: <5 mm Hg @ 68 °F

VAPOR DENSITY (Air=1): >5

PERCENT VOLATILE BY WEIGHT (%): 100

SPECIFIC GRAVITY OR BULK DENSITY: 0.85 @ 60/60 °F

SOLUBILITY IN WATER: Negligible (<0.1%)

EVAPORATION RATE (BuAc = 1): <0.01%

APPEARANCE and ODOR: Light brown liquid, banana odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: >230 °F (PMCC)

FLAMMABILITY LIMITS IN AIR (% V): UEL: 7.0	LEL: 0.9

EXTINGUISHING MEDIA: Water spray (fog), dry chemical, foam or carbon dioxide

SPECIAL FIRE FIGHTING PROCEDURES: Use water to keep fire-exposed containers cool. If a leak or spill has not ignited, use water spray to disperse the vapors and to provide protection for men attempting to stop the leak. Water spray may be used to flush spills away from exposures. Minimize breathing gases, vapor, fumes or decomposition products. Use supplied air breathing equipment for enclosed or confined spaces or as otherwise needed.

UNUSUAL FIRE & EXPLOSION HAZARDS: None

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidants such as liquid chlorine, concentrated oxygen, sodium or calcium hypochlorites.

HAZARDOUS DECOMPOSITION: Fumes, smoke, carbon monoxide, sulfur oxides, aldehydes and other decomposition products in the case of incomplete combustion.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: NA

HEALTH HAZARDS: Eyes: May cause eye irritation. Skin: Prolonged or repeated skin contact may cause irritation and dermatitis. Inhalation: Due to low vapor pressure, inhalation is unlikely. Ingestion: May cause narcosis.

CHRONIC EFFECTS OF OVER EXPOSURE: NA

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Flush with water while removing contaminated clothing and shoes. Wash with soap and water. Do not reuse clothing or shoes until cleaned, discard oil soaked leather shoes and garments. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. If irritation persists, obtain medical attention.

INGESTION: DO NOT induce vomiting. If vomiting occurs spontaneously keep head below hips to prevent aspiration into lungs. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Normally not needed at ambient temperatures. Use NIOSH approved organic vapor cartridge respirator with dust, fumes and mist. Prefilter where PEL limitation is exceeded.

VENTILATION: Use local exhaust to capture vapor, mists or fumes if necessary. Provide ventilation sufficient to prevent exceeding recommended exposure limit or buildup of explosive concentrations of vapor in air.

PROTECTIVE CLOTHING: Apron or other impervious clothing as needed to avoid contaminating regular clothing.

EYE PROTECTION: Chemical or splash proof goggles

PROTECTIVE GLOVES: Neoprene or equivalent resistant gloves

OTHER PRECAUTIONS: Safety shower and eye wash station should be available.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating. Product is readily removed by waterless hand cleaners before washing with soap and water.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Recover free product. Add sand, earth or other suitable absorbent to spill area and collect in containers for disposal. Minimize skin contact. Open all windows and doors. Keep product out of sewers and watercourses by diking or impounding. Advise authorities if product has entered or may enter sewers, watercourses or extensive land areas.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store in a cool, dry area. Follow all label directions. Keep container closed when not in use. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than