MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 1/8/02										RUTH-LESS

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-467

TRADE NAME: RUTH-LESS

SYNONYMS: Heavy Duty Floor Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3, Fire: 0, Reactivity: 3, Special Hazard: None

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Sodium Hydroxide			1310-73-2		10-15			2 mg/m3			2 mg/m3			Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: ~60 lbs/ ft3

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Yellow, Powder

ODOR: Pine odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Material does not support combustion. Use media suitable for surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Do not use water on material itself. If large quantities of combustible materials are involved, use water in flooding quantities as spray or fog. Material is corrosive, use adequate personal protection. Avoid breathing dusts and fumes from burning material.

UNUSUAL FIRE & EXPLOSION HAZARDS: None known. Addition of water generates heat.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Prolonged contact with aluminum, tin, lead, and zinc may produce hydrogen gas

CHEMICAL INCOMPATIBILITY: Acids, soft metals

HAZARDOUS DECOMPOSITION: Carbon dioxide and hydrogen gas upon acid or metal contact

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin

HEALTH HAZARDS: Skin: Corrosive. Causes destruction and irreversible harm to human tissue. Eyes: Corrosive. Causes destruction and irreversible harm to eyes. Inhalation: Dusts are corrosive and can lead to respiratory irritation or burns. Constrictive scarring is possible.

CHRONIC EFFECTS OF OVER EXPOSURE: Burns to skin and eyes. Respiratory irritation. Dermatitis and irritation of skin due to defatting. Ingestion leads to vomiting.

MEDICAL CONDITIONS AGGRAVATED: Burns, dermatitis, respiratory irritation

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Immediately flood with large quantities of water. Seek physician if burned.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. Dilute by drinking water. Seek medical attention immediately.

INHALATION: Move to fresh air and eliminate dusts.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Wear a NIOSH/MSHA approved dust mask

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: Chemical goggles plus full face shield

PROTECTIVE GLOVES: Chemical resistant gloves

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly after using this material.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Ventilate the area, wear proper protective equipment. Sweep up material and return to container or discard. Rinse away residue with water. Keep material out of water sources and sewers. Adding water to the spilled material leads to heat generation and possible splattering. Avoid all environmental contamination.

WASTE DISPOSAL METHOD: According to all applicable pollution and environmental regulations for alkaline, corrosive solids.

PRECAUTIONS IN HANDLING: Keep out of the reach of children. Keep container closed when not in use, material will absorb moisture from the air. Store in a cool, dry area. Read complete cautions and directions before use. Do not reuse empty container. Do not use with any other chemicals or cleaners. Do not inhale dusts. Avoid all skin and eye contact. Use appropriate protective equipment when handling broken or leaking containers. Use common sense and good housekeeping as with any corrosive material.

NA = Not Available     <= Less than     >= Greater than