Latest Revision Date: 7/28/98										WOODY

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-471

TRADE NAME: WOODY

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 2, Reactivity: 0, Special: O

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Mix of predominantly saturated hydrocarbons	647428-97		~100			NA			NA			No

Benzene				71-43-2		0.01-0.09			10 ppm			10 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 321-373 °F

VAPOR PRESSURE: 2.7 mm Hg @ 20 °C

VAPOR DENSITY (Air=1): 4.83

PERCENT VOLATILE BY WEIGHT (%): 100

SPECIFIC GRAVITY OR BULK DENSITY: 0.7781

SOLUBILITY IN WATER: Negligible

APPEARANCE: Clear, colorless liquid

ODOR: Petroleum odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 105 °F

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: According to NFPA Guide: use dry chemical, foam or carbon dioxide.

SPECIAL FIRE FIGHTING PROCEDURES: Water may be ineffective on flames, but should be used to cool fire-exposed containers. If a leak or spill has not ignited, use water spray to disperse the vapors and to provide protection for personnel attempting to stop the leak.

UNUSUAL FIRE & EXPLOSION HAZARDS: DANGER! Flammable materials may release vapors that travel a considerable distance to a source of ignition and flashback. Containers may explode in a fire. Do not expose to heat, sparks, flame or other sources of ignition. Explosive air-vapor mixtures may form.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: NA

Conditions To Avoid: Toxic levels of carbon monoxide, carbon dioxide, irritating aldehydes and ketones

CHEMICAL INCOMPATIBILITY: Reacts violently with strong oxidizers.

HAZARDOUS DECOMPOSITION: NA

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation

HEALTH HAZARDS: Eyes: May cause irritation. Skin: May cause irritation. Inhalation: Harmful. May cause dizziness and drowsiness. Aspiration hazard if swallowed (can enter lungs and cause damage). Combustible liquid and vapor. This product contains benzene. Prolonged or repeated exposure to benzene has been associated with anemia and leukemia in humans.

CHRONIC EFFECTS OF OVER EXPOSURE: Repeated skin contact may cause a persistent irritation or dermatitis.

MEDICAL CONDITIONS AGGRAVATED: Repeated skin contact may aggravate an existing dermatitis because of the products irritating properties.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. Remove contaminated clothing. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: Seek medical attention immediately. Only induce vomiting as directed by a doctor. Never give anything by mouth to an unconscious or convulsing person.

INHALATION: Move to fresh air. If not breathing, give artificial respiration. If breathing is difficult, qualified personnel may administer oxygen. Get immediate medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If vapor, mist or dust is generated, use respirator approved by MSHA/NIOSH as appropriate. Supplied air respiratory protection should be used for cleaning large spills or upon entry into tanks, vessels or other confined spaces.

VENTILATION: Adequate to meet component occupational exposure limits.

PROTECTIVE CLOTHING: Uniforms, coveralls or lab coats should be worn. Launder or dry-clean when soiled. Exposed workers should wash exposed skin several times daily with soap and water.

EYE PROTECTION: Avoid contact. Chemical goggles should be worn. Do not wear contact lenses.

PROTECTIVE GLOVES: Gloves resistant to chemicals and petroleum distillates should be worn. Exposed workers should wash exposed skin several times daily with soap and water.

OTHER PRECAUTIONS: Exposure limit for Total Product: for standard solvent, 100 ppm (TWA ACGIH and OSHA).

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Exposed workers should wash exposed skin several times daily with soap and water.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Ventilate area. Avoid breathing vapor. Use self-contained breathing apparatus or supplied air for large spills or confined areas. Contain spill if possible. Wipe up or absorb with suitable material and shovel. Prevent entry into sewers and waterways. Avoid contact with skin, eyes or clothing.

WASTE DISPOSAL METHOD: If discarded in its present form, it would have the hazardous waste number D018 and D001 respectively. Dispose of according to Federal, State and Local authorities.

PRECAUTIONS IN HANDLING: Store away from heat and open flame. A placard is required only when material is contained in packaging or container that exceeds 110 gallons, or in tank car or tank truck. Transport, handle and store in accordance with OSHA Regulation 1910.106 and applicable DOT Regulations.

NA = Not Available     <= Less than     >= Greater than