MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/24/98										IRON MAN

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-510

TRADE NAME: IRON MAN

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 0, Fire: 0, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 219 °F

pH: 3.8

VAPOR PRESSURE: 23.7 mm Hg

VAPOR DENSITY(Air=1): 1.0

PERCENT VOLATILE BY WEIGHT (%): >60

SPECIFIC GRAVITY OR BULK DENSITY: 1.19

SOLUBILITY IN WATER: Soluble

EVAPORATION RATE (BuAc = 1): >1.0

APPEARANCE: Very dark brown liquid

ODOR: Woody odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water, foam, carbon dioxide, dry chemical

SPECIAL FIRE FIGHTING PROCEDURES: Fire fighters should be equipped with full protective gear including self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: None

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: None

CHEMICAL INCOMPATIBILITY: None

HAZARDOUS DECOMPOSITION: Thermal decomposition may produce oxides of carbons.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Ingestion

HEALTH HAZARDS: NA

CHRONIC EFFECTS OF OVER EXPOSURE: Eyes: Mild irritation. Ingestion: Nausea, diarrhea.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: Give affected person several glasses of water and induce vomiting. Seek medical attention immediately. Never give anything by mouth to an unconscious person.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Use with adequate ventilation

VENTILATION: Local exhaust: Not required		Mechanical: Not required

PROTECTIVE CLOTHING: NA

EYE PROTECTION: Chemical goggles

PROTECTIVE GLOVES: Rubber gloves

OTHER PRECAUTIONS: Not required

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Pick up absorbent material or flush away with water.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store in a cool, dry place. Keep container tightly closed when not in use. KEEP OUT OF REACH OF CHILDREN.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.