MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 6/12/01										SLAM

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-552

TRADE NAME: SLAM

SYNONYMS: Super Impact Grease

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 1, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Petroleum Oil Mixture		8002-05-9		NA			5 mg/m3			5 mg/m3			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: NA

VAPOR DENSITY (Air=1): 12

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 0.8

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Dark green gel

ODOR: Oil odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: >400°F (COC)

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Dry chemical (preferred), carbon dioxide, foam, water

SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus

UNUSUAL FIRE & EXPLOSION HAZARDS: Empty containers may retain product residue including flammable or explosive vapors. Do not cut, drill, grind or weld on or near full, part full or empty drums.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Oxidizing atmosphere

CHEMICAL INCOMPATIBILITY: Chlorine or oxygen

HAZARDOUS DECOMPOSITION: Thermal: Unidentified organics		Oxidation: Oxides of carbon

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Skin

HEALTH HAZARDS: Inhalation: Massive exposure to vapors or fumes may cause headaches, dizziness and/or drowsiness. Eyes: May cause irritation. Skin: May cause irritation. Ingestion: May cause vomiting, nausea and diarrhea.

CHRONIC EFFECTS OF OVER EXPOSURE: None noted

MEDICAL CONDITIONS AGGRAVATED: Dermatitis

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritation persists.

INGESTION: DO NOT induce vomiting. Give 3-4 glasses of water. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None required

VENTILATION: None required in excess to normal ventilation

PROTECTIVE CLOTHING: Oil resistant apron

EYE PROTECTION: If splashing is to occur, wear splash goggles or safety glasses.

PROTECTIVE GLOVES: If skin contact will be made, wear oil resistant gloves.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly after handling.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Stop flow, dike spill and eliminate all ignition sources. Absorb on inert material and scrape up.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Avoid prolonged exposure to mist and vapors. Avoid flames, sparks and hot surfaces. Keep containers closed. Shelf-life of one year. KEEP OUT OF REACH OF CHILDREN. Empty containers may contain explosive gas.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.