MATERIAL SAFETY DATA SHEET						COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053							1700 SOUTH FOURTH STREET

Information #: (402) 379-0100							NORFOLK, NE 68701

Latest Revision Date: 7/29/98											SUPERIOR

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-600

TRADE NAME: SUPERIOR

SYNONYMS: Rubberized Crack Filler

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 0, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Asphalt				8052-42-4		40-45			5 ppm (ceil)		5 ppm(ceil)		Yes

Proprietary Elastomer		9003-55-8		1-2			2.2 mg/m3		2.2 mg/m3		No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212 °F

pH: NA

VAPOR PRESSURE: 25 mm Hg @ 70 °F

VAPOR DENSITY: >1

PERCENT VOLATILE BY WEIGHT (%): 40-45

SPECIFIC GRAVITY OR BULK DENSITY: 1.0 ± 0.2 @ 60 °F

SOLUBILITY IN WATER: Miscible

EVAPORATION RATE (BuAc = 1): Slower

APPEARANCE: Brown to gray viscous material

ODOR: Slight thiol odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Since this is a water based material, it will not burn until all moisture has been removed. If fire should occur at extreme elevated temperatures, use foam, CO2 or dry chemical extinguishers.

SPECIAL FIRE FIGHTING PROCEDURES: Fire fighters should wear full protective equipment and self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: Water sprayed on burning material may cause frothing, steam and eruptions.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Since this is a water based emulsion, settlement will occur. The settlement rate is dependent on the dilution and storage temperature.

CHEMICAL INCOMPATIBILITY: This material is incompatible with acids. DO NOT heat above 212 °F. Any and all additives should be evaluated before use in the field.

HAZARDOUS DECOMPOSITION: Irritating or toxic vapors may be released when this material is burned.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Ingestion, Eyes

HEALTH HAZARDS: Inhalation: Vapors which are released during the curing process are not known to contain hazardous materials. However, residual monomers are present in the product and overexposure to monomer vapor could result in respiratory irritation and central nervous system depression. Ingestion: Could cause burns in the mouth, esophagus and stomach. Skin: May cause irritation and itching. Eyes: May cause inflammation, irritation and possible corneal burns. Documentation indicates that contact lenses SHOULD NOT be worn when exposed to this product.

CHRONIC EFFECTS OF OVER EXPOSURE: See Health Hazards.

MEDICAL CONDITIONS AGGRAVATED: Any pre-existing skin disorders could possibly be aggravated by exposure to this product.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Vapors which are released during the curing process are not known to contain hazardous material. However, if breathing is difficult, move to fresh air. Seek qualified or specialized medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Approved MSHA/NIOSH particulate and organic vapor canister respirators as regulated by 29 CFR 1910.134.

VENTILATION: Normal exterior environmental applications eliminate the need for specialized additional ventilation. However, application of material should be made downwind from applicator.

PROTECTIVE CLOTHING: Rubber footgear is preferred over leather boots or tennis shoes. All items saturated with product should be disposed of properly.

EYE PROTECTION: Goggles or face shield

PROTECTIVE GLOVES: Flannel lined neoprene and nitrile gloves are recommended.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: If possible, stop the source of the leak. Dike and contain to eliminate environmental contamination. For small spills, squeegee into thin layer, cover with dirt or aggregate. If water pollution occurs, notify police, local EPA and the National Response Center (1-800-424-8802).

WASTE DISPOSAL METHOD: If disposal is necessary, contact your state environmental agency for guidance as to proper methods and receiving locations of waste materials in your area.

PRECAUTIONS IN HANDLING: Keep from freezing. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.