MATERIAL SAFETY DATA SHEET						COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053							1700 SOUTH FOURTH STREET

Information #: (402) 379-0100							NORFOLK, NE 68701

Latest Revision Date: 7/29/98										BLACK TOP

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-601

TRADE NAME: BLACK TOP

SYNONYMS: Cold Asphalt Mix

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 1, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Asphalt				8052-42-4		5-6			5 mg/m3			5 mg/m3			NA

Kerosene				8008-20-6		1-2			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: >340°F

VAPOR PRESSURE: Very low

VAPOR DENSITY: >5.0

PERCENT VOLATILE BY WEIGHT (%): Negligible

SPECIFIC GRAVITY OR BULK DENSITY: 1.0±0.2 @ 60°F

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE: Black, viscous granular solid

ODOR: Petroleum odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: >200°F

EXTINGUISHING MEDIA: Material will not burn unless preheated. Dry chemical or foam extinguishers are preferred. Material contains rocks which could shatter due to heat or sudden source of water.

SPECIAL FIRE FIGHTING PROCEDURES: Material will not burn unless preheated. DO NOT enter confined firespace without full bunker gear which includes a positive pressure self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: Elevated temperatures and sources of ignition should be avoided to prevent accidental fires. Material contains rocks which could shatter due to heat or sudden source of water. Containerized materials would allow concentration of ignitable vapors to collect above material.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Avoid elevated temperatures and sources of ignition.

CHEMICAL INCOMPATIBILITY: Asphalt will dissolve from mixture if gasoline or diesel fuel is spilled on it.

HAZARDOUS DECOMPOSITION: Thermal decomposition could produce hydrogen sulfide, carbon monoxide, carbon dioxide, sulfur dioxide, airborne solids, particulates, gases and other unidentified compounds.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Ingestion, Skin

HEALTH HAZARDS: Inhalation: Asphalt fumes which would be generated at high temperatures cause irritation to the nose, throat and respiratory tract of animals. Ingestion: High viscosity oils, i.e. asphalt, have been known to be less toxic when ingested than lower viscosity higher volatile oils. Skin: Serious burns could result in direct contact with heated material. Heated material contains rocks and sand which retain heat. After material has cooled, exposure to material can cause dermatitis and oil acne. Eyes: Serious burns could result from direct contact with heated material.

CHRONIC EFFECTS OF OVER EXPOSURE: Prolonged exposure may possibly increase the risk of more serious respiratory or skin disorders.

MEDICAL CONDITIONS AGGRAVATED: Any pre-existing skin or respiratory disorder could possibly be aggravated by exposure to this product. Skin effects include irritation, dermatitis and oil acne.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: If skin contact occurs when material is hot, immerse the area in cold water to assist in cooling and reduce discomfort of the affected areas. DO NOT use solvents to remove asphalt. To remove, use waterless hand cleaner. If burning or irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain immediate ophthalmological assistance. This material contains sand which could cause ophthalmological damage.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: In confined areas, care must be taken to assure that TLV/TWA limits are not exceeded. Approved MSHA/NIOSH particulate and organic vapor canister respirators or a positive atmosphere supplied respirator as regulated by 29 CFR 1910.134 should be used if TLV/TWA limits are exceeded.

VENTILATION: Normal exterior application should not require the need for mechanical ventilation.

PROTECTIVE CLOTHING: Contaminated clothing should be cleaned before reuse. Leather items, i.e. boots and gloves, which are saturated should be disposed of properly.

EYE PROTECTION: Goggles or face shield

PROTECTIVE GLOVES: Flannel-lined neoprene or nitrile gloves are recommended.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: If material enters a waterway, notify police, local EPA and the National Response Center (1-800-424-8802).

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store in original container. Read all label directions before using product. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.