MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 2/16/99											SWAT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-650

TRADE NAME: SWAT

SYNONYMS: Insecticide

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3, Fire: 1, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

O,O-Dimethyldithiophosphate		1217-55-5		57.00			10 ppm			10 ppm			No

of ethyl mercaptosuccinate (Malathion)

Heavy Aromatic Naptha		64741-98-6	37.00			300 ppm			300 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 183°F

VAPOR PRESSURE: 0.004 mm Hg

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY VOLUME (%): >30

SPECIFIC GRAVITY OR BULK DENSITY: 1.083

SOLUBILITY IN WATER: Emulsifies

EVAPORATION RATE (BuAc = 1): >1.0

APPEARANCE: Amber liquid

ODOR: Strong mercaptan odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 157°F (closed cup)

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Foam, carbon dioxide, dry chemical

SPECIAL FIRE FIGHTING PROCEDURES: Firefighters must be equipped with full protective gear including self-contained breathing apparatus. Fire may produce hazardous vapors. Minimize amount of water used to fight fire as run-off may cause pollution. Contain run-off for proper disposal.

UNUSUAL FIRE & EXPLOSION HAZARDS: Cool drums in vicinity of fire with water mist to prevent explosion. Solvent vapors may flash back.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: High temperatures

CHEMICAL INCOMPATIBILITY: Avoid contamination with water, acids or alkalies.

HAZARDOUS DECOMPOSITION: Thermal decomposition may yield oxides of carbon and sulfur.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Ingestion

HEALTH HAZARDS: Excessive salivation, respiratory difficulty, muscle twitch, convulsions, exhaustion, headache, blurred vision, constriction of pupils (acetylcholine effects).

CHRONIC EFFECTS OF OVER EXPOSURE: See Health Hazards

MEDICAL CONDITIONS AGGRAVATED: See Health Hazards

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Remove contaminated clothing and launder before re-use. Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: Call physician or poison control center immediately. DO NOT induce vomiting. Never give anything by mouth to an unconscious person.

INHALATION: Move to fresh air. Administer artificial respiration if indicated and call a physician immediately.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Use with adequate protection. Use a NIOSH approved respirator with organic vapor filter and pesticide pre-filter to prevent breathing of mists and vapors.

VENTILATION: Local Exhaust: Recommended	Mechanical: Recommended

EYE PROTECTION: Chemical goggles

PROTECTIVE GLOVES: Rubber gloves

OTHER PRECAUTIONS: Protective clothing, emergency shower and eye wash station

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Contain spill. Soak up on inert absorbent material and place in closed container for disposal.

WASTE DISPOSAL METHOD: Consult local environmental authorities. Refer to NACA “Disposal Methods For Organic Phosphates And Related Materials”.

PRECAUTIONS IN HANDLING: Store in a cool, dry place. Do not contaminate water, food feed, or seed by storage or disposal. Wash thoroughly after usage. Keep container tightly closed when not in use. Do not contaminate any body of water by cleaning of equipment. Keep out of reach of children. Do not breathe vapors.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.