MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/27/98										THE HEAT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-700

TRADE NAME: THE HEAT

SYNONYMS: Granular Sewer Solvent

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 4, Fire: 1, Reactivity: 3, Special Hazard: None

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Sodium Hydroxide			1310-73-2		70-80			2 mg/m3			2 mg/m3			No

Sodium Nitrate			7631-99-4		13-20			NA			NA			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 1.2

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Red, beaded solid

ODOR: No odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Material will not burn.

SPECIAL FIRE FIGHTING PROCEDURES: Generates large amounts of heat upon contact with water.

UNUSUAL FIRE & EXPLOSION HAZARDS: Water contact generates large amounts of heat and noxious gases.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with acids, acid solutions, soft metals and moisture

CHEMICAL INCOMPATIBILITY: Acids and their solutions

HAZARDOUS DECOMPOSITION: Moisture generates heat and pungent, irritating fumes.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation

HEALTH HAZARDS: Skin: Highly corrosive, causing burns. Eyes: Highly corrosive, causing blindness. Inhalation: Dusts may be irritating to the respiratory tract. Fumes generated during use are irritating. Ingestion: Leads to burns.

CHRONIC EFFECTS OF OVER EXPOSURE: Immediately and severely corrosive to skin and eyes. Dusts and fumes irritating to the respiratory tract.

MEDICAL CONDITIONS AGGRAVATED: Bronchial irritation from dusts or fumes.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. Remove contaminated clothing. If irritation persists, seek a physician.

EYES: Flush with water for at least 30 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. Drink large amounts of water. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: To avoid breathing fumes generated by use of this product.

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: To protect skin, eyes and clothing from contact.

EYE PROTECTION: Chemical workers goggles, full face shield

PROTECTIVE GLOVES: Required - rubber

OTHER PRECAUTIONS: Use with caution - highly corrosive and heat generating.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly after using or handling this product.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Immediately wear appropriate protective clothing; shovel up material and return to container. Final residues may be flushed with copious amounts of water. Keep spilled material out of waterways and sewers.

WASTE DISPOSAL METHOD: According to local pollution ordinances for strongly alkaline, corrosive solids.

PRECAUTIONS IN HANDLING: Keep out of reach of children. Keep container closed when storing. Avoid all skin and eye contact. Store in a cool, dry area away from acids. Protect contents from moisture. Do not add water to the container. Never use with acid-based drain cleaners. Use in ventilated areas, avoid breathing fumes generated while using this product. Read complete label for use, cautions and directions before using.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.