MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/27/98										DECLOG

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-702

TRADE NAME: DECLOG

SYNONYMS: Non Acid Drain Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3, Fire: 0, Reactivity: 1, Equipment: D

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Potassium Hydroxide 45%		1310-58-3		10-20			2 mg/m3			2mg/m3			Yes

Sodium Hydroxide			1310-73-2		20-30			2 mg/m3			2mg/m3			Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212°F

pH: 14

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 1.27

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE:  Red liquid

ODOR: No odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Not flammable. Use extinguishing method appropriate for surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Wear full protective clothing.

UNUSUAL FIRE & EXPLOSION HAZARDS:  Extinguish all nearby sources of ignition in spill situations since flammable hydrogen gas may be liberated on contact with some metals, especially aluminum.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Excessive heat and contamination of any kind

CHEMICAL INCOMPATIBILITY: Acids, combustible materials, and metals such as aluminum, tin, galvanized zinc, brass and bronze. Also avoid many organic chemicals, especially nitrocarbons, leather and wool, and organic acids and their anhydrides. This product may react with various sugars to  form hazardous carbon monoxide.

HAZARDOUS DECOMPOSITION: Generates hazardous mists at 218-288°F

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Ingestion, Skin, Inhalation

HEALTH HAZARDS: Causes chemical burns.

CHRONIC EFFECTS OF OVER EXPOSURE: Information on chronic health hazards not currently available.

SIGNS AND SYMPTOMS OF EXPOSURE: Irritation, burning of skin, mucous membranes and respiratory symptoms.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Flush with running water for at least 30 minutes. OBTAIN MEDICAL ATTENTION if irritation persists..

EYES: Immediately flush with cool running water for at least 30 minutes. CONTACT A PHYSICIAN IMMEDIATELY.

INGESTION: DO NOT INDUCE VOMITING. If conscious, give large amounts of water or milk. CONTACT A PHYSICIAN IMMEDIATELY.

INHALATION: Move to fresh air. Give artificial respiration if not breathing. SEEK MEDICAL ATTENTION.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Not required in normal use.

VENTILATION: Local exhaust sufficient, mechanical exhaust not required.

PROTECTIVE CLOTHING: Impervious clothing to prevent contact

EYE PROTECTION: Chemical goggles or full face shield

PROTECTIVE GLOVES: Rubber or neoprene gloves

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly after using or handling this product.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: ALWAYS WEAR PROTECTIVE CLOTHING WHEN CONTROLLING A SPILL SITUATION, ESPECIALLY EYE PROTECTION. Contain spilled product and neutralize with diluted hydrochloric acid. Neutralized product should be disposed of in accordance with existing Federal, State and Local ordinances.

WASTE DISPOSAL METHOD: Dispose of in accordance with existing Federal, State or Local pollution regulations.

PRECAUTIONS IN HANDLING: Practice common sense in the handling of these products. Avoid spills and splashing of opened products. Store product in tightly closed original container. Store in cool dry place. Provide adequate ventilation. KEEP OUT OF REACH OF CHILDREN. Keep product from freezing.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.