MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 3/18/98										DEFOAMER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-704

TRADE NAME:  DEFOAMER

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Flammability: 2, Reactivity: 2, Special Hazard: None

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Butyl Cellosolve			111-76-2		10-15%			50 ppm			50 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: Range 300 - 580°F

VAPOR PRESSURE: No data

VAPOR DENSITY: >1

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 0.82 - 0.84

SOLUBILITY IN WATER:  Emulsifiable

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE and ODOR:  Red liquid, petroleum odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  120 - 165°F (TOC)

FLAMMABILITY LIMITS IN AIR (% V): LEL: ~1.3     UEL: ~6

EXTINGUISHING MEDIA: Agents approved for Class B hazards: e.g. dry chemical, carbon dioxide, halogenated agents, foam, steam and water fog

SPECIAL FIRE FIGHTING PROCEDURES: Fire fighters should use self-contained breathing apparatus. Water may be used to keep adjacent containers cool, but may be unsuitable as an extinguishing media. Do not spread burning liquid with water used for cooling purposes.

UNUSUAL FIRE & EXPLOSION HAZARDS:  Vapor is heavier than air and may travel a considerable distance to ignition and flashback. Combustible liquid: Keep away from ignition sources.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with ignition sources such as open flames, sparks, got-glowing surfaces or electric arcs

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents, strong acids or bases, selected amines

HAZARDOUS DECOMPOSITION: Combustion leads to formations of carbon monoxide and carbon dioxide.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin

HEALTH HAZARDS: Eye and skin irritant, may cause dermatitis. Harmful or fatal if swallowed. Vapors are irritating to respiratory tract. High vapor concentrations lead to central nervous system depressions, nausea, vomiting and eventual internal organ damage.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye, skin or respiratory tract irritation, dermatitis, depression, nausea, vomiting

MEDICAL CONDITIONS AGGRAVATED: Dermatitis, bronchial conditions

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. Seek physician for persistent irritation.

EYES: Flush with water for at least 15 minutes rotating eyeball. Seek a physician.

INGESTION: DO NOT induce vomiting. Seek immediate medical attention.

INHALATION: If vapor concentration is high, seek fresh air; if necessary, administer artificial respiration and seek medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Required if vapor concentrations are high.

VENTILATION: Local exhaust: May be required if vapor concentrations are high.

PROTECTIVE CLOTHING: To prevent wetting of skin or clothes

EYE PROTECTION: If splashing is to occur, wear safety glasses, goggles or face shield.

PROTECTIVE GLOVES: If skin contact is to be made, wear impermeable gloves.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke when using this product. Wash hands after use.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Remove all potential ignition sources; provide adequate ventilation. Flush spilled material into adequate retaining area or container with water. Small spills may be collected with an absorbent. Prevent the spill and water flushings from reaching natural waterways and public water systems and sewers.

WASTE DISPOSAL METHOD: According to applicable Federal, State, County and Local pollution and waste disposal ordinances. Care must be taken to avoid environmental contamination.

PRECAUTIONS IN HANDLING: Keep out of reach of children Keep container closed when not in use. Store in a cool place away from ignition sources and strong oxidizing agents. Read all label directions and cautions before use. Use with adequate ventilation. Avoid all skin and eye contact. Contaminated clothing should be removed immediately and washed before reuse. Avoid vapor inhalations and high concentration of vapors. Material is combustible and must be handled as any other combustible liquid. Do not expose empty drums to fire, sparks, etc. as residual vapors may ignite.

NA = Not Applicable     <= Less than     >= Greater than     ~= Approximately