MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/27/98										COUNT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-709

TRADE NAME: COUNT

SYNONYMS: Enzymes

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 1, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Viable Bacterial Cultures		116469-86-4	100			NA			NA			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: 6.5-8.5

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): 0.00

SPECIFIC GRAVITY OR BULK DENSITY: NA

SOLUBILITY IN WATER: Moderate - contains viable bacteria

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Free flowing, tan-brown powder

ODOR: Yeast-like odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water spray, carbon dioxide, dry chemical

SPECIAL FIRE FIGHTING PROCEDURES: None

UNUSUAL FIRE & EXPLOSION HAZARDS: NA

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable under normal conditions

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Strong acids or alkali compounds may inactivate biological cultures.

HAZARDOUS DECOMPOSITION: None

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: NA

HEALTH HAZARDS: Ingestion: No effect if ingested in small amounts. Eyes: This material may cause eye irritation. Skin: Slight redness on hands and forearms if individual has a history of dermal allergic reaction. Inhalation: Product may produce nuisance dust which will irritate respiratory passages.

CHRONIC EFFECTS OF OVER EXPOSURE: Dermatitis and skin sensitization can develop after repeated and/or prolonged contact with human skin.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Remove contaminated clothing and footwear. Wash clothing and footwear before reuse. Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If needed, use MSHA/NIOSH approved respirator for nuisance dust.

VENTILATION: General mechanical ventilation.

PROTECTIVE CLOTHING: Take all precautions to prevent skin contact.

EYE PROTECTION: Safety glasses recommended if used in a manner that creates dust.

PROTECTIVE GLOVES: NA

OTHER PRECAUTIONS: As determined or recommended by Federal, State and Local regulations.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Wear approved respirator for nuisance dust during cleanup. For small spills, flush to waste treatment sewer (product is biodegradable). For large spills, contain and collect for reuse.

WASTE DISPOSAL METHOD: Disposal of this product or its residues must be in accordance with Federal, State and Local authorities.

PRECAUTIONS IN HANDLING: No TLV assigned to this mixture. Control of exposure to below the TLV for the ingredients will be sufficient. Minimize exposure in accordance with good hygiene practice. Avoid handling which would create large amounts of dust. Use approved respirator for nuisance dust. To maintain shelf life, avoid prolonged exposure to high temperatures and humidity.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.