MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/28/98										SEWER AID

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-712

TRADE NAME: SEWER AID

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3, Fire: 0, Reactivity: 2, Special: X

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Sulfuric Acid 66° Baume		7664-93-9		93.2			1 mg/m3			1 mg/m3			Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 535 °F

pH: <1.00

VAPOR PRESSURE: <0.300 @ 77 °F

VAPOR DENSITY (Air=1): 3.4

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 1.835 @ 68 °F

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): 1

APPEARANCE: Colorless or clear amber liquid

ODOR: No odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: Non-flammable

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Dry chemical or carbon dioxide

SPECIAL FIRE FIGHTING PROCEDURES: Stay upwind if product is involved in a fire. Wear protective clothing. Use water spray to cool containers exposed to fire. Water mixed with product evolves heat and may cause splattering.

UNUSUAL FIRE & EXPLOSION HAZARDS: Contact with organic materials can cause spontaneous ignition. Contact with metals can evolve hydrogen, a poisonous gas.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Extreme heat

CHEMICAL INCOMPATIBILITY: Metals/metal blends, hydrocarbons, inorganic bases, organic acids, organic bases, water. Contact with some metals can generate explosive hydrogen gas.

HAZARDOUS DECOMPOSITION: Extreme temperatures may release sulfur dioxide.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: This product can cause chronic health hazards and can cause damage to all body parts.

CHRONIC EFFECTS OF OVER EXPOSURE: Inhalation: Can cause burns to the respiratory tract. Ingestion: Material will burn mouth, throat and stomach. Eyes: Will cause serious burning and may cause blindness.

MEDICAL CONDITIONS AGGRAVATED: Chronic exposure of this product may result in erosion of the teeth and mouth inflammation.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. Remove contaminated clothing. Call a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. Give egg whites and water to dilute. Seek medical attention immediately.

INHALATION: Move to fresh air. If breathing has stopped, administer artificial respiration. Get medical attention immediately.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None required in normal use.

VENTILATION: Local exhaust: Sufficient		Mechanical: NIOSH approved

PROTECTIVE CLOTHING: Rubber apron and shoes are recommended.

EYE PROTECTION: Chemical goggles or face shield

PROTECTIVE GLOVES: Rubber or neoprene

OTHER PRECAUTIONS: Safety shower, eyewash station and washing facilities should be readily available.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Use protective equipment to avoid contact with skin or eyes. Neutralize spill with soda ash.

WASTE DISPOSAL METHOD: Dispose of in accordance with Federal, State and Local pollution regulations.

PRECAUTIONS IN HANDLING: Practice common sense in handling this product. Store product in original, tightly closed container. Keep away from combustible materials. Store in well vented areas. Keep out of the reach of children. Read all labels for directions and precautions.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.