MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 4/8/98											APPLAUSE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-757

TRADE NAME: APPLAUSE

SYNONYMS: Instant Hand Sanitizer

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Reactivity: 0, Flammability: 3, Health: 1, Personal Protection: A

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Ethyl Alcohol			64-17-5		60			1000 ppm			1000 ppm			Yes

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg:  180°F

pH: 6.8 - 7.5

VAPOR PRESSURE: Not established

VAPOR DENSITY: NA

PERCENT NONVOLATILE BY WEIGHT (%): 1 2

SPECIFIC GRAVITY OR BULK DENSITY: 0.895±0.015

SOLUBILITY IN WATER:  Appreciable

EVAPORATION RATE (BuAc = 1): 1.2 (until surface film is formed)

APPEARANCE:  Clear liquid

ODOR: Citrus

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  68°F

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Class A, B, or C; water spray, carbon dioxide, foam

SPECIAL FIRE FIGHTING PROCEDURES: Standard fire fighting procedures may be followed, including full protective gear, NIOSH approved self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: None known

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: None known

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents

HAZARDOUS DECOMPOSITION: Combustion will produce oxides of carbon, nitrogen and sulfur.

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: NA

OXIDIZER:  NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Ingestion

HEALTH HAZARDS: Eyes: Contact may cause irritation. Ingestion: May cause stomach cramps or diarrhea.

CHRONIC EFFECTS OF OVER EXPOSURE: Irritation or burning sensation

MEDICAL CONDITIONS AGGRAVATED: None known

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with water. If irritation persists, seek physician.

EYES: Flush immediately with water for 15 minutes. Contact physician.

INGESTION: Rinse mouth with water. Then feed two glasses of water and induce vomiting. Contact physician.

INHALATION: NA

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Not required

VENTILATION: Not required

PROTECTIVE CLOTHING: NA

EYE PROTECTION: Safety glasses for bulk handling

PROTECTIVE GLOVES: Not required

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Follow OSHA laws and normal housekeeping practices.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Contain spill with an absorbent and collect for disposal. Rinse area with water.

WASTE DISPOSAL METHOD: Dispose in accordance with Federal, State and Local regulations. Rinse container before recycling it.

PRECAUTIONS IN HANDLING: Store in tightly closed container away from heat and oxidizing agents. Keep out of reach of children. Keep from freezing.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.