MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/31/98								SOLVENT CLEANER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-801

TRADE NAME: SOLVENT CLEANER

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 0, Fire: 2, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Hydrotreated Heavy Naphtha		64742-48-9	NA			100 ppm			100 ppm			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 315-394°F

VAPOR PRESSURE mm Hg @ 20 °C: 1.6 mm Hg @ 20°C

VAPOR DENSITY (Air=1): 3.5

PERCENT VOLATILE BY WEIGHT (%): 100 @ 1 atm and 25°C

SPECIFIC GRAVITY OR BULK DENSITY: 0.80

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (BuAc = 1): <0.1

APPEARANCE: Clear, colorless liquid

ODOR: NA

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 104 °F Method: TCC Note: Minimum

AUTOIGNITION TEMPERATURE: ~490 °F

FLAMMABILITY LIMITS IN AIR (% V): LEL: 1.3	UEL: 9.8 @ 77 °F

EXTINGUISHING MEDIA: Foam, dry chemical or water spray

SPECIAL FIRE FIGHTING PROCEDURES: Use water spray to cool fire exposed surfaces and to protect personnel. Isolate "fuel" supply from fire. Avoid spraying water directly into storage containers due to danger of boilover. This liquid is volatile and gives off invisible vapors. Either the liquid or vapor may settle in low areas or travel some distance along the ground or surface to ignition sources where they may ignite or explode.

UNUSUAL FIRE & EXPLOSION HAZARDS: NA

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents

HAZARDOUS DECOMPOSITION: NA

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: Slightly irritating but does not injure eye tissue. Skin: Frequent or prolonged contact may irritate and cause dermatitis. Inhalation: High vapor/aerosol concentrations (> ~ 1000 ppm) are irritating to the eyes and the respiratory tract. May cause headaches, dizziness, anesthesia, drowsiness, unconsciousness and other central nervous system effects including death. Ingestion: Small amounts of this product aspirated into the respiratory system during ingestion or vomiting. May cause mild to severe pulmonary injury, possibly progressing to death.

CHRONIC EFFECTS OF OVER EXPOSURE: NA

MEDICAL CONDITIONS AGGRAVATED: Dermatitis

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. Remove grossly contaminated clothing including shoes and launder before reuse. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritation persists.

INGESTION: DO NOT induce vomiting. Keep at rest. Seek medical attention immediately.

INHALATION: Using proper respiratory protection, immediately remove the affected victim from exposure. Administer artificial respiration if breathing has stopped. Keep at rest. Seek medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: NIOSH/MSHA approved respirators may be necessary to prevent overexposure by inhalation.

VENTILATION: The use of mechanical dilution ventilation is recommended whenever this product is used in a confined space, is heated above ambient temperatures or is agitated.

PROTECTIVE CLOTHING: Appropriate clothing to avoid wetting of the skin.

EYE PROTECTION: Safety glasses with side shields

PROTECTIVE GLOVES: Chemical resistant gloves

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Eliminate sources of ignition. Prevent additional discharge of material if possible to do so without hazard. Keep public away and advise authorities. Prevent liquid from entering sewers, waterways or low areas. Contain spilled liquid with sand or earth. Do not use combustible materials. Recover by pumping (use an explosion proof or hand pump) or with a suitable absorbent.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store away from any source of ignition. Keep container closed when product is not being used. Read and follow all label directions.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.