MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/27/98								POOL FLOCCULENT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-854

TRADE NAME: POOL FLOCCULENT

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 0, Fire: 0, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212 °F

pH: 4.2

VAPOR PRESSURE: NA

VAPOR DENSITY: NA

PERCENT VOLATILE BY VOLUME (%): >60

SPECIFIC GRAVITY OR BULK DENSITY: 1.05

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): 1

APPEARANCE: Clear to colorless heavy liquid

ODOR: Odorless

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water, foam, carbon dioxide and dry chemical

SPECIAL FIRE FIGHTING PROCEDURES: Wear protective clothing. Fire fighting apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: None

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: None

CHEMICAL INCOMPATIBILITY: None

HAZARDOUS DECOMPOSITION: Thermal decomposition may produce oxides of carbons.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Ingestion

HEALTH HAZARDS: Ingestion: Harmful or fatal if swallowed.

CHRONIC EFFECTS OF OVER EXPOSURE: Eyes: Mild irritation. Skin: Prolonged contact may lead to irritation. Ingestion: Gastrointestinal irritation, nausea and vomiting.

MEDICAL CONDITIONS AGGRAVATED: None known

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritation persists.

INGESTION: Give person several glasses of water and induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Use with adequate ventilation.

VENTILATION: Local exhaust: Yes		Mechanical: No

PROTECTIVE CLOTHING: NA

EYE PROTECTION: If splashing is to occur, wear chemical goggles.

PROTECTIVE GLOVES: If hand contact is to be made, wear rubber or neoprene gloves.

OTHER PRECAUTIONS: Emergency shower or eyewash station

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Soak up spill with an inert absorbent.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store in cool, dry area. Keep container closed when not in use. Keep out of reach of children. Read all directions on label before using.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.