MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/31/98										LOWER IT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-856

TRADE NAME: LOWER IT

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 0, Reactivity: 1, Special Hazard: None

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Sodium Bisulfate			7681-38-1		94.5			15 mg/m3			10 mg/m3			No

Sodium Sulfate			7757-82-6		5.0			15 mg/m3			10 mg/m3			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 350 °F

VAPOR PRESSURE: None

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 2.72

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE and ODOR: White, granular material, Odorless

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: Non-flammable

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Use any appropriate medium for extinguishing surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Fire fighters should wear self-contained breathing apparatus and full protective clothing. Use water spray to cool nearby containers and structures exposed to fire.

UNUSUAL FIRE & EXPLOSION HAZARDS: Sodium bisulfate dissolves in water to form sulfuric acid and may release oxides of sulfur in a fire.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Moisture, temperatures above 572 °F

CHEMICAL INCOMPATIBILITY: Avoid contact with strong alkaline materials such as caustic. Reacts with water to form weak sulfuric acid solutions. Do not mix with liquid chlorine, bleach or similar products.

HAZARDOUS DECOMPOSITION: Carbon monoxide, carbon dioxide, oxides of sulfur

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Ingestion, Inhalation, Skin, Eyes

HEALTH HAZARDS: Inhalation: Breathing dust may irritate the nose, throat, mucous membranes and respiratory tract. Eyes: Dust will irritate the eyes and prolonged contact may burn or damage the eyes. Skin: Dust may irritate the skin. Moisture on the skin, such as perspiration, may cause burns due to the formation of sulfuric acid. Ingestion: Swallowing the dusts or solids can be severely irritating to the mouth and gastrointestinal tract due to the formation of sulfuric acid. Nausea, vomiting and abdominal pain may result.

CHRONIC EFFECTS OF OVER EXPOSURE: NA

MEDICAL CONDITIONS AGGRAVATED: Skin disorders, respiratory system disorders

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. Remove contaminated clothing and shoes; wash before reuse. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. If conscious, give lots of water or milk. Follow with milk of magnesia. Seek medical attention immediately.

INHALATION: Move to fresh air. Give artificial respiration if not breathing. Call a physician.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If use conditions generate dusts, wear a dust mask.

VENTILATION: Local mechanical exhaust ventilation capable of minimizing dust emissions at the point of use

PROTECTIVE CLOTHING: Long-sleeved shirt, trousers, safety shoes, rubber gloves and rubber apron

EYE PROTECTION: Safety glasses or goggles

PROTECTIVE GLOVES: Rubber

OTHER PRECAUTIONS: An eyewash station and safety shower should be nearby and ready for use.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Wear protective equipment. For small spills, sweep up and dispose of in DOT approved waste containers. Keep out of sewers, storm drains, surface waters and soil.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store in a cool, dry, well-ventilated place. Store away from all other chemicals and potential sources of contamination. Keep container tightly closed when not in use. Do not use pressure to empty containers. Wash thoroughly after handling. Do not get in eyes, on skin or clothing. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately