MATERIAL SAFETY DATA SHEET						COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053							1700 SOUTH FOURTH STREET

Information #: (402) 379-0100							NORFOLK, NE 68701

Latest Revision Date: 7/31/98									TRICHLORO MAXI STICKS

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-857

TRADE NAME: TRICHLORO MAXI STICKS

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3, Fire: 0, Reactivity: 2, Special Hazard: X

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Trichloro-s-triazinetrione		NA		NA			15 mg/m3			10 mg/m3			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: Decomposes @ 225-230 °C (437-446 °F)

pH: 3.0

VAPOR PRESSURE: NA

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

BULK DENSITY: 56 lbs/ft3 (regular); 57 lbs/ft3 (granular and extra granular)

SOLUBILITY IN WATER: 1%

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: White, crystalline solid

ODOR: Slight chlorine odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: Decomposes @ 225 °C (437-446 °F)

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: A thermal decomposition can be extinguished by flooding with copious quantities of water or by isolating the decomposing material and allowing it to be consumed.

SPECIAL FIRE FIGHTING PROCEDURES: Fire fighters and others subject to products of decomposition should wear full protective clothing and self-contained breathing apparatus. Chlorine-containing gases with traces of phosgene can be liberated at temperatures in excess of 400 °F.

UNUSUAL FIRE & EXPLOSION HAZARDS: Small quantities of water react with this chlorination composition to form hazardous quantities of nitrogen trichloride which is violently explosive. Strong oxidizers and contact with organic matter may cause fires. Will burn with evolutions of chlorine and other toxic gases. Fire: If possible, isolate container in open air or well-ventilated area. Flood a large volume of water.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Chlorine-containing gases can be produced. Traces of phosgene can be liberated at high temperatures above 400 °F.

CHEMICAL INCOMPATIBILITY: Water, oxidizable organic materials, ammonia, urea and other nitrogen-containing compounds; inorganic reducing compounds; calcium and lithium hypochlorite; alkalis

HAZARDOUS DECOMPOSITION: NA

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: Eyes: Causes eye damage. Skin: Causes burns to the skin. Inhalation: Causes irritation of nose and throat. Ingestion: Harmful if swallowed.

CHRONIC EFFECTS OF OVER EXPOSURE: Dermal contact and inhalations are expected to be the primary routes of occupational exposure and is considered to be corrosive to the eyes and skin. Exposure to trichloro-s-triazinetrione dust fumes has been reported to produce eye, nose, throat and respiratory tract irritation.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Immediately brush off excess chemical and wash with soap and water. Remove contaminated clothing; wash before reuse. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: Drink milk, water or raw egg whites. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Use NIOSH/MSHA approved equipment with full face piece when airborne exposure limits are exceeded. Consult respirator manufacturer to determine appropriate equipment type for given location.

VENTILATION: Local exhaust: Provide ventilation to control levels below airborne exposure limits.

PROTECTIVE CLOTHING: Impervious protective clothing. Provide safety showers where skin contact can occur.

EYE PROTECTION: Wear chemical safety goggles to prevent eye contact. Have eyewash stations immediately available.

PROTECTIVE GLOVES: Impervious gloves: rubber or plastic

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Contain spilled material. KEEP SPILLED MATERIAL DRY. Do not allow water or other incompatible substances to come into contact with spilled material. Keep this product out of sewers, watersheds and water systems. Sweep, scoop or vacuum up all spilled material, contaminated soil and other contaminated materials and place in clean, dry containers for disposal. Complete clean up on a dry basis.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Read all labels. Keep product tightly closed when not in use. Store in a dry place. Do not allow water to get into container. Keep containers off wet floors.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately