MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 8/6/98											SODA ASH

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-858

TRADE NAME: SODA ASH

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 0, Reactivity: 0, Special Hazard: X

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Sodium Carbonate			497-19-8		100			15 mg/m3			10 mg/m3			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 752°F

pH: >11.4

VAPOR PRESSURE: NA

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: >2.5 @ 68°F

SOLUBILITY IN WATER: 23%

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Powder

ODOR: No odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Media appropriate for surrounding fire

SPECIAL FIRE FIGHTING PROCEDURES: Firefighters should wear full face, self-contained breathing apparatus and impervious protective clothing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Dust can be an explosion hazard. Prevent buildup of dust on walls, floors and equipment.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Inorganic acids

HAZARDOUS DECOMPOSITION: Carbon monoxide, carbon dioxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Ingestion, Inhalation

HEALTH HAZARDS: Eyes: May cause irritation and burns. Skin: May cause irritation and burns. Inhalation: May cause irritation. Ingestion: May cause irritation.

CHRONIC EFFECTS OF OVER EXPOSURE: NA

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. Rinse mouth with water. Drink water to dilute. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Use NIOSH/MSHA approved respirator if TLV limits are exceeded.

VENTILATION: Adequate ventilation is required to minimize exposure or to maintain exposure levels below OSHA/ACGIH requirements. Local mechanical ventilation may be required.

PROTECTIVE CLOTHING: NA

EYE PROTECTION: Chemical goggles

PROTECTIVE GLOVES: Impervious

OTHER PRECAUTIONS: Safety shower, eyewash station and washing facilities should be readily available.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Sweep, shovel or pick up and return to dry, clean container.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Keep container closed when not in use. Store in a cool, dry place. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.