MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/31/98								GRANULAR CHLORINE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-859

TRADE NAME: GRANULAR CHLORINE

SYNONYMS: Calcium Hypochlorite

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): NA

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Calcium Hypochlorite		NA		65			NA			NA			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 356 °F

pH: Alkaline

VAPOR PRESSURE: NA

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 0.98

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: White granular or white tablets

ODOR: Slight chlorine odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: Non-flammable

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water only

SPECIAL FIRE FIGHTING PROCEDURES: Firefighters must wear self-contained breathing apparatus with full face piece.

UNUSUAL FIRE & EXPLOSION HAZARDS: Releases oxygen gas; containers may rupture.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Excessive heat

CHEMICAL INCOMPATIBILITY: Acids or combustible materials

HAZARDOUS DECOMPOSITION: Any form of contamination

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: Eyes: Corrosive; contact with even a minute amount for a short period can cause severe irritation and even blindness.

CHRONIC EFFECTS OF OVER EXPOSURE: Burning or irritation to the eyes, throat, mouth or skin

MEDICAL CONDITIONS AGGRAVATED: None known

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting; drink large quantities of water and vegetable oil. Seek medical attention immediately.

INHALATION: Move to fresh air. If breathing stops, administer artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: NIOSH approved

VENTILATION: Local exhaust: Not accepted		Mechanical: NIOSH approved

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: Goggles

PROTECTIVE GLOVES: Rubber

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Use extreme caution in handling spilled material. Sweep up visible material using a clean, dry shovel and dissolve material in water. Can be neutralized with hydrogen peroxide.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store in a cool, dry area. Keep container closed at all times while not in use. Store away from any combustible material. Keep out of reach of children and follow all directions.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.