MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 8/7/98									CHLORINE - TABLETS

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-860

TRADE NAME: CHLORINE - TABLETS

SYNONYMS: Calcium Hypochlorite Tablets

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): NA

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Calcium Hypochlorite		NA		65			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 356°F

pH: Alkaline

VAPOR PRESSURE: NA

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: ~0.98

SOLUBILITY IN WATER: 217 g/L @ 81°F

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: White tablets

ODOR: Slight chlorine odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Water only: smothering is ineffective as product supplies oxygen.

SPECIAL FIRE FIGHTING PROCEDURES: NIOSH/MSHA approved, pressure demand self-contained breathing apparatus with full face piece for possible exposure to hazardous gases.

UNUSUAL FIRE & EXPLOSION HAZARDS: Decomposes at 356°F, releasing oxygen gas; containers may rupture.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Unstable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Acids, combustible materials, organics, reducing agents. Any form of contamination (especially combustible materials or excessive heat may cause decomposition or fire).

HAZARDOUS DECOMPOSITION: Acids or ammonia contamination or excessive heat will result in the release of oxygen gas.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Ingestion, Inhalation

HEALTH HAZARDS: Eyes: Corrosive, contact with even a minute amount for a short period can cause severe irritation and even blindness. Skin: Contact can cause severe irritation, burns or tissue destruction. Inhalation: Dust can cause irritation or burns to the nose, throat and mucous membranes. Respirable dust reaching the lungs is extremely toxic and can be quickly fatal. Ingestion: Causes severe burns to the digestive tract and can be fatal.

CHRONIC EFFECTS OF OVER EXPOSURE: NA

MEDICAL CONDITIONS AGGRAVATED: None known

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air. If breathing is difficult, provide oxygen. Give artificial respiration if not breathing. Get medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If dusty conditions are encountered, use NIOSH/MSHA approved respirator with acid gas cartridge and dust pre-filter.

VENTILATION: None, unless dusty conditions are encountered.

PROTECTIVE CLOTHING: To prevent skin contact

EYE PROTECTION: Chemical safety goggles

PROTECTIVE GLOVES: Rubber gloves

OTHER PRECAUTIONS: Safety shower, eyewash station

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Sweep up all visible material using a clean, dry shovel and broom and dissolve material in water.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Keep in original container in a cool, dry place. Keep container closed when not in use. Do not mix with any other chemicals. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.