MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 3/4/99										AQUA COAT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-862

TRADE NAME: AQUA COAT

SYNONYMS: Swimming Pool Paint

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 2, Reactivity: 0

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Mineral Spirits			8052-41-3		40-50			100 ppm			100 ppm			Yes

Zinc				1314-13-2		5			10 ppm			10 ppm			No

Amorphous Silica			58855-54-9	1-5			6 mg/m3			10 mg/m3			No

Talc				14807-96-6	10-15			3 mg/m3			2 mg/m3			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 316 °F

pH: NA

VAPOR PRESSURE: NA

VAPOR DENSITY (Air=1): Lighter than air

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 1.22

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (BuAc = 1): Slower than ether

APPEARANCE: Blue or white liquid

ODOR: Aliphatic odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 103 °F

FLAMMABILITY LIMITS IN AIR (% V): UEL: 6      LEL: 1

EXTINGUISHING MEDIA: Foam, CO2, dry chemical

SPECIAL FIRE FIGHTING PROCEDURES: Full protective equipment and self-contained breathing apparatus should be used. Water may be used to cool closed containers to prevent pressure build-up; possible auto-ignition or explosion from heating.

UNUSUAL FIRE & EXPLOSION HAZARDS: Handle as an ignitable liquid. Keep containers tightly closed and isolate from heat, electrical equipment, sparks or flame.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Excessive heat, all possible sources of ignition, poor ventilation, corrosive atmospheres, excessive aging

CHEMICAL INCOMPATIBILITY: Alkaline materials, strong oxidizing materials, strong acids

HAZARDOUS DECOMPOSITION: Thermal decomposition or combustion can produce fumes containing organic acids, carbon dioxide and carbon monoxide.

HAZARDOUS POLYMERIZATION: Will not occur under normal conditions.

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: Inhalation: Solvent vapor or mist can cause dizziness, breathing difficulty, headaches, irritation, tearing, discomfort, redness and blurred vision. Skin: Can cause irritation. Ingestion: Can cause mouth, throat, esophagus and stomach irritation, nausea, vomiting and diarrhea. Eyes: Can cause irritation.

CHRONIC EFFECTS OF OVER EXPOSURE: Can cause defatting of skin which can lead to dermatitis. Reports have associated repeated or prolonged occupational overexposure with permanent brain and nervous system damage.

MEDICAL CONDITIONS AGGRAVATED: Pre-existing eye, skin, liver, kidney and respiratory disorders

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water while removing contaminated clothing and shoes. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting! Contact a physician or your local poison control center immediately.

INHALATION: Move to fresh air. If breathing stops, apply artificial respiration and seek medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Do not breathe vapors or spray mist. Wear an appropriate, properly fitted respirator (NIOSH/MSHA approved) during the use of this product until vapor and mists are exhausted unless air monitoring demonstrates vapor and mist levels are below applicable exposure limits.

VENTILATION: Provide general clean air dilution or local exhaust ventilation in volume and pattern to keep the air contaminant concentrations below the lower explosion limit and applicable exposure limits.

PROTECTIVE CLOTHING: Avoid contact with contaminated clothing. Wash contaminated clothing, including shoes, before reuse.

EYE PROTECTION: Avoid contact with eyes. Use safety eye wear with splash guards or side shields, chemical goggles or face shields.

PROTECTIVE GLOVES: Use chemical/solvent impermeable gloves to avoid contact with product.

OTHER PRECAUTIONS: Provide eyewash station and emergency shower. Use of protective creams, head caps, etc. is recommended.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Eliminate all sources of ignition (flames, hot surfaces, and electrical, static or frictional sparks). Avoid breathing vapors. Ventilate area. Contain and remove with inert absorbent and non-sparking tools. Keep out of sewers.

WASTE DISPOSAL METHOD: Collect absorbent/spilled liquid into metal containers. Dispose of in accordance with Federal, State and Local regulations.

PRECAUTIONS IN HANDLING: Keep away from excessive heat, sparks or open flames. Keep containers closed when not in use. Store in cool, well ventilated, approved areas. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.