MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/31/98						CONDENSATE LINE CONDITIONER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-905

TRADE NAME: CONDENSATE LINE CONDITIONER

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): NA

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212 °F

pH: NA

VAPOR PRESSURE: 25-30 mm Hg @ 20 °C

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): 8.73

SPECIFIC GRAVITY OR BULK DENSITY: 1.0

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): Slower than butyl acetate

APPEARANCE: Water-white liquid

ODOR: Amine odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Material will not support combustion.

SPECIAL FIRE FIGHTING PROCEDURES: If involved in a fire, material will not burn but may liberate noxious morpholine vapors.

UNUSUAL FIRE & EXPLOSION HAZARDS: None

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Acids and acid solution contact

CHEMICAL INCOMPATIBILITY: Acids and their solutions

HAZARDOUS DECOMPOSITION: Morpholine vapors

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin

HEALTH HAZARDS: Eyes: May cause irritation or burns. Skin: Possibly absorbed through the skin. May cause irritation or burns. Inhalation: Irritating vapors. Ingestion: Harmful if swallowed.

CHRONIC EFFECTS OF OVER EXPOSURE: Skin and eye irritation or burns, irritation to the respiratory tract

MEDICAL CONDITIONS AGGRAVATED: Dermatitis, bronchial conditions

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: To avoid breathing vapors

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: Recommended - safety glasses

PROTECTIVE GLOVES: Recommended - rubber

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Ventilate the area; collect the spill with an absorbent. Neutralize the residue with a weak acid; collect or rinse away the residue.

WASTE DISPOSAL METHOD: According to local pollution ordinances. Do not dump into sewers or waterways.

PRECAUTIONS IN HANDLING: Keep out of reach of children. Keep container closed when not in use. Store in a cool place away from acids. Read entire label for complete cautions and directions. Alkaline liquid is absorbable through the skin so wear appropriate clothing. Clean contaminated clothing before reuse.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.