MATERIAL SAFETY DATA SHEET						COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053							1700 SOUTH FOURTH STREET

Information #: (402) 379-0100							NORFOLK, NE 68701

Latest Revision Date: 7/31/98								ALL PURPOSE BOILER COMPOUND

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-906

TRADE NAME: ALL PURPOSE BOILER COMPOUND

SYNONYMS: NA

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 3, Flammability: 0, Reactivity: 1, Personal Protection: D

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Sodium Hydroxide			1310-73-2		NA			2 mg/m3			2 mg/m3			No

Sodium Sulfite			775-83-7		NA			NA			NA			No

Sodium Hexa Meta Phosphate		NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 212°F

pH: NA

VAPOR PRESSURE: 25-30 mm Hg @ 20°C

VAPOR DENSITY: >1

PERCENT VOLATILE BY WEIGHT (%): 0

SPECIFIC GRAVITY OR BULK DENSITY: 1.05 - 1.10

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE: Cloudy liquid

ODOR: Faint non-descript odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Material will not support combustion; use media suitable for combating surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Containers involved in a fire may be cooled by applying flooding quantities of water from as far a distance as possible. Use appropriate safety equipment in combating a fire involving this material. If containers are leaking avoid all skin and eye contact. Do not inhale fumes, vapors, smoke or combustion products.

UNUSUAL FIRE & EXPLOSION HAZARDS: Overheated containers may melt or rupture. Cool as necessary.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Contact with acids

CHEMICAL INCOMPATIBILITY: Acids and soft metals. Do not mix directly with other chemicals.

HAZARDOUS DECOMPOSITION: None known

HAZARDOUS POLYMERIZATION: Will not occur

CORROSIVE TO METAL: NA

OXIDIZER: NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Ingestion

HEALTH HAZARDS: Eyes: Irritant. Skin: Irritant. Ingestion: Harmful or fatal if swallowed. The alkaline nature of this material may lead to eye damage, skin burns and internal organ damage.

CHRONIC EFFECTS OF OVER EXPOSURE: Eye and skin irritation or burns, dermatitis

MEDICAL CONDITIONS AGGRAVATED: Dermatitis

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Remove contaminated clothing. Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush well with copious amounts of water for at least 15 minutes, seek a physician immediately.

INGESTION: Dilute by drinking large volumes of water or fruit juices. Seek physician. DO NOT induce vomiting.

INHALATION: Move to fresh air. If breathing has stopped, give artificial respiration. Get medical attention immediately.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Usually not required

VENTILATION: Local exhaust: Fresh air

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: Safety glasses and face shield

PROTECTIVE GLOVES: Rubber

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke when using this material. Wash hands after use.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Wear appropriate protective equipment for alkaline compounds. Stop the flow of material if possible. Contain the spill, if necessary, by diking the area. Collect the spill with an absorbent or by wet vacuuming. Clean residue by flushing with water. Keep the spill out of waterways, sewers and water sources. Avoid all environmental contamination.

WASTE DISPOSAL METHOD: According to all applicable environmental and pollution ordinances. Dispose of the empty container in accordance with RCRA regulations.

PRECAUTIONS IN HANDLING: Keep out of reach of children. Keep container closed when not in use. Read entire label for complete cautions and directions. Do not use or mix with other chemicals. Store in a cool area but protect from freezing. Do not reuse empty containers. Use with appropriate personal protective equipment. Avoid all eye and skin contact. Contaminated clothing must be removed before reuse. Do not handle leaking containers without proper protective equipment. Use normal safety precautions and common sense when dealing with this or any alkaline material.

NA= Not Applicable     <=Less than     >=Greater than